ATDRTABL.DOC 	04/24/87	D. J. Wilke

Table of drive types for AT and compatibles.  Use this to configure the

CMOS RAM chip when running the AT advanced diagnostics software.  Determine

your drive type from the table and answer with the appropriate number when

queried from the setup program.  There are 47 entries in the BIOS ROM

within the AT, with some entries reserved for future expansion.  The BIOS

address for the start of the table is F000:E401H in an IBM AT.

DRIVE	CYL	HEADS	WRITE		CTL	LANDING		SECTORS

TYPE			PRECOMP		BYTE	ZONE		PER TRACK

===========================================================================

01	306	4	128		0	305		17

02	615	4	300		0	615		17

03	615	6	300		0	615		17

04	940	8	512		0	940		17

05	940	6	512		0	940		17

06	615	4	NONE		0	615		17

07	462	8	256		0	511		17

08	733	5	NONE		0	733		17

09	900	15	NONE		8	901		17

10	820	3	NONE		0	820		17

11	855	5	NONE		0	855		17

12	855	7	NONE		0	855		17

13	306	8	128		0	319		17

14	733	7	NONE		0	733		17

15	-	-	R E S E R V E D	-	-	-	-

16	612	4	ALL CYLINDERS	0	663		17

17	977	5	300		0	977		17

18	977	7	NONE		0	977		17

19	1024	7	512		0	1023		17

20	733	5	300		0	732		17

21	733	7	300		0	732		17

22	733	5	300		0	732		17

23	306	4	ALL CYLINDERS	0	336		17

24	-	-	R E S E R V E D	-	-	-	-

25	-	-	R E S E R V E D	-	-	-	-

26	-	-	R E S E R V E D	-	-	-	-

27	-	-	R E S E R V E D	-	-	-	-

28	-	-	R E S E R V E D	-	-	-	-

29	-	-	R E S E R V E D	-	-	-	-

30	-	-	R E S E R V E D	-	-	-	-

30	-	-	R E S E R V E D	-	-	-	-

32	-	-	R E S E R V E D	-	-	-	-

33	-	-	R E S E R V E D	-	-	-	-

34	-	-	R E S E R V E D	-	-	-	-

35	-	-	R E S E R V E D	-	-	-	-

36	-	-	R E S E R V E D	-	-	-	-

37	-	-	R E S E R V E D	-	-	-	-

38	-	-	R E S E R V E D	-	-	-	-

39	-	-	R E S E R V E D	-	-	-	-

40	-	-	R E S E R V E D	-	-	-	-

41	-	-	R E S E R V E D	-	-	-	-

42	-	-	R E S E R V E D	-	-	-	-

43	-	-	R E S E R V E D	-	-	-	-

44	-	-	R E S E R V E D	-	-	-	-

45	-	-	R E S E R V E D	-	-	-	-

46	-	-	R E S E R V E D	-	-	-	-

47	-	-	R E S E R V E D	-	-	-	-

#############################