Recent reviews of communications programs in PC Magazine and elsewhere

failed to test the performance of the various programs under the stress

of line disturbances, slow computers, or background operation.  To

remedy this situation, here are the results of Omen Technology's

Protocol Torture Tests run from time to time on various file transfer

protocols and programs.

Anyone who questions the honesty or validity of these tests is invited

to send a knowledgeable representative to Omen to monitor a repeat of

the test(s) in question, and/or submit newer released versions of

software for retest.  The test conditions are clearly described and easy

to reproduce for the benefit of those who wish to repeat the tests.

Chuck Forsberg

503-621-3406

Omen Technology INC

17505-V NW Sauvie IS RD

Portland OR 97231

Protocol Stress Tests 7-5-87

Omen Technology is connected to a rural telephone exchange in Sauvie

Island, a suburb of Portland Oregon.  This variability of phone line

quality has led to an emphasis on protocol robustness.  The Omen

Technology Protocol Stress Test simulates noise patterns seen on

marginal telephone calls remarkably closely, given the random nature

of line noise.

Sending Computer (TeleGodzilla BBS system): IBM PC, V-20, 8087,

Maynard 10 MB hard disk, IBM CGA, B&W composite monitor, AST MegaPlus

II multifunction board.  2400 bps modems.

Receiving Computer: QIC Labs AT Clone, 8mHz 0ws, Seagate 4051,

Genoa Super EGA, Thompson UltraScan monitor.

The receiving modem was dialed into the sending modem (TeleGodzilla).

Both lines were on the same exchange, and line hits in this

configuration are rare.  Line noise was generated by a Radio Shack

Duophone 145 connected in parallel with the receiving modem, pulse

redialing the number 1-800-123-4567.  This generates "line hits"

simulating those seen on marginal phone connections.

There has been some discussion about the relevance of such a test setup

and the distribution of "line hits" it generates.  Line hits at short

periodic intervals of less than a second or so (generated by clock

slippage) are not well simulated, and only protocols with an extremely

short minimum block length ( << 128) can function in such environments.

This test is not that demanding; properly written XMODEM or XMODEM

based protocols succeed most of the time, and the protocols that have the

reputation among the bulletin boards of reliable operation under stress

rarely fail this test.

The test file used in these tests was BBS_TEE.ARC (a BBS program that

operates under Pro-YAM or ZCOMM), selected for its convenient 29960 byte

length.  It is available for downloading from TeleGodzilla, but any .ARC

file of about that length should give the same results.

Software:

YAMK version 16.74 (ZMODEM, YMODEM), GT PowerComm Version 1221 (MEGAlink)

Each computer was running the same communications software.  No TSRs

were present.

Test#   PROT    Throughput Remarks

1       MEGALI     Failed    RX reported success at block 136

2       MEGALI     Failed    TX reported success at block 40, RX slow to abort

3       MEGALI     Failed    TX reported success at block 95

4       MEGALI      44 cps   Redials exhausted before transfer finished

5       MEGALI     Failed    TX reported success at block 88

1       YMODEM               Transfer successful, throughput not recorded

2       YMDM-k      86 cps   Transfer finished before redials exhausted

1       ZMODEM     122 cps   (z pt30 on RX) Transfer finished first

2       ZMODEM      91 cps   (z pt20 on RX) Transfer finished first

1       SK          82 cps   Pro-YAM to Pro-YAM Transfer finished first

2       SK          94 cps   (rx: k pt2) Pro-YAM to Pro-YAM XFER finished first

3       ZMODEM     120 cps   (z pt20 pW600 on RX) Transfer finished first

1       SK          50 cps   XTALK Mark IV to X4 Transfer finished first

Throughputs were reported by the receiving program. SK=Kermit Sliding Windows

Protocol Stress Tests Performed 12-13-87

Configuration: As above except AT clone had a CGA clone board instead of

EGA clone.  Same BBS_TEE.ARC file.  As before, transfers were at 2400 bps.

Software:

YAM.EXE 17.02, Crosstalk Mark IV 1.01, GT Power 13.00, Procomm 2.42.  As

of 12-14-87 these are believed to be the latest released versions of

these programs.  A new Procomm was downloaded from Compuserve to verify

the correctness of the PROCOMM.EXE file.  YMODEM is a batch protocol,

called YMODEM Batch on Procomm and Crosstalk Mark IV.  Timings by stopwatch.

S/W	PROT	Transfer Time / Remarks

Pro-YAM	ZMODEM	3:53	Tight timing: 2 sec, 1 sec within packets

Pro-YAM	ZMODEM	3:59	Tight timing: 2 sec, 1 sec within packets

Pro-YAM	ZMODEM	3:39	Tight timing: 2 sec, 1 sec within packets

Pro-YAM	ZMODEM	3:54	Tight timing: 2 sec, 1 sec within packets

Pro-YAM	ZMODEM	4:54	Standard timing

Pro-YAM	ZMODEM	4:36	Standard timing

Pro-YAM	ZMODEM	5:03	Standard timing

Pro-YAM	Kermit	6:14	Standard timing

Pro-YAM	YMODEM	FAILED	Tight timing: 2 sec, 1 sec within packets

Pro-YAM	YMODEM	5:44	Standard timing

Pro-YAM	YMODEM	5:20	Standard timing

Pro-YAM	YMDM-k	6:07	Standard timing

Pro-YAM	YMDM-k	5:06	Standard timing

GT Pwr	Megali	8:30

GT Pwr	Megali	9:30

GT Pwr	Megali	FAILED TX reported success

GT Pwr	Megali	FAILED TX reported success

GT Pwr	Megali	10:23

Procm	YMDM	FAILED Stack overflow, lockup

Procm	YMDM	FAILED Stack overflow, lockup

XTALKm4	DART	9:84

XTALKm4	DART	5:06

XTALKm4	DART	4:15

XTALKm4	DART	4:16

XTALKm4	DART	5:59

XTALKm4	DART	5:54

XTALKm4	DART	5:08

XTALKm4	DART	FAILED RX:Too many Errors TX: locked up

XTALKm4	DART	4:45

XTALKm4	DART	5:11

XTALKm4	YMDM	FAILED

XTALKm4	YMDM	FAILED

XTALKm4	YMDM	FAILED

XTALKm4	YMDM	2:13 Normal transfer time without line hits

XTALKm4	YMDM	FAILED

XTALKm4	YMDM	FAILED