An Introduction to Classical Dynamics Simulation of Atomic Collisions at Surfaces

M.A. Karolewski

ã M.A. Karolewski, 1997.

karolewski@alum.mit.edu

Version 1.0, released June 1997.

The author permits the reproduction and distribution of this work without restriction. This article is distributed with the Simulation Kit (a software suite for classical dynamics simulation of atomic collisions). News about updates to the Simulation Kit (if any) will be carried at the following URL during 1997:

http://www.geocities.com/CapeCanaveral/Lab/7039/simkit.htm

1. Introduction

1.1. Objectives

The first purpose of this article is to cover the minimum of background theory and concepts needed by anyone who wishes to use classical dynamics simulation to model the behaviour of atomic collision systems. The range of particle energies considered here is 1-10,000 eV. This energy range typically covers secondary ion mass spectrometry (SIMS) and ion scattering spectroscopy (ISS) as well as a number of less familiar techniques and processes. The discussion begins with a review of the theory of the binary collision, and very quickly moves on to the task of computer implementation of many-particle simulations. The treatment given here is more superficial and less rigorous than what is found in the standard texts, since its objective is to get the reader going on the simulations as quickly as possible.

The article’s second purpose is to serve as a manual of demonstration projects for the author’s Simulation Kit, and as a repository for other information that seemed out of place in an on-line Help system. To a large extent, the discussion throughout the article is built around the features offered by the Simulation Kit. Ideally, you should browse through this article before, or while, you attempt a serious evaluation of the Simulation Kit. Some of the diagrams are in colour, and may be better viewed on-screen than in a printed version.

This article is written under severe time constraints, and I cannot pretend that it is anywhere near complete. I hope that later in 1997 I will be able to release a revised version with more worked examples in it. In the meantime, I would appreciate it if readers could point out any errors of fact or omission, or suggest ways in which the presentation could be improved.

1.2. Sources

A number of references to the primary and review literature are supplied in context throughout the article. Since I anticipate that some readers of this article will be research students from a variety of backgrounds, I have also given references to textbooks I have used profitably myself over the years for some general topics that may not have taken root during undergraduate days. The references may be found at the end of the article.

There are several texts and review articles which cover the central subject matter of this article especially well, which should be mentioned at the outset. These are by: Eckstein, Mashkova and Molchanov, Harrison, Smith and Harrison, Robinson and Niehus et al. The reader should get hold of as many of these key references as possible.

1.3. Notational Conventions

Vector quantities are written in boldface (r) whereas scalar quantities are represented by an italic font (r), or sometimes plain font (r). I use b for the impact parameter (designated as p or s by some writers).

1.4. Simulation Kit Components

There is frequent reference throughout this article to the following computer programs: Spider, Snook and Winnow. Spider is a utility whose function is to design and generate the input data files used for the simulations. The simulation engine itself resides in the Snook program. Winnow, finally, is the program which processes the output data created by Snook, and turns this data into scientifically useful information (sputter yields, energy spectra etc.). It may be helpful to new users of Winnow to regard the Snook output files as database files; the purpose of Winnow, then, is to process user-defined queries to this database, and to present the information in an acceptable form.

2. Classical Scattering Theory

2.1. Classical Scattering Theory

The classical scattering theory described in undergraduate textbooks deals predominantly with the binary collision, involving a two-body, radial potential, V(r12), in which two interacting particles first approach each other, and later recede. (Good graduate-level text-book treatments of scattering can be found in Goldstein, or in Landau and Lifshitz.) The strength of the interaction depends only on the separation (r12) of the two particles. The scattering terminology applies to forces which have a finite range and tend to zero as the separation increases. However, the decline of the potential with separation need not be monotonic.

More complex interaction potentials can be envisaged. For example, the interaction might depend on the relative orientations of two interacting objects (e.g. a point charge interacting with a fixed dipole), or it might depend on the relative orientations of three point particles (a so-called ‘three-body’ potential): potentials involving three (or more) bodies are required to rationalise the structures of small clusters in terms of potential theory, for example. The force associated with the interaction of charged particles with magnetic fields depends upon the velocity of the particle, and not just its position in the field: this fact is exploited in mass spectrometry, for example.

For the most part, the collisions discussed in this chapter (and in this article) will be elastic collisions. An elastic collision is one in which the sum of the kinetic energies of the particles after the collision, E1 + E2, is the same as it was before the collision, E0 (see Fig. 1). Inelastic collisions are those in which energy is transferred to some internal mode of excitation (e.g. electronic excitations) or dissipated (e.g. by friction). Chapter 4 will offer some comments on the effects and simulation of inelastic processes in atomic collision systems.

The purpose of the following review is not to work through the equations of classical scattering theory (which can be found in any mechanics textbook), but to summarise the objectives and achievements of the classical theory which are useful in the context of atomic collisions.

2.2. Kinematics of Binary Collision

The classical scattering problem can be analysed at several different levels.

At the first level, the kinematic analysis involves the application of conservation laws (energy, momentum) to the problem, and establishes what relationships hold between respectively the collisional energy transfer, the scattering angles and the relative masses of the participating particles (see Fig. 1 for explanation of symbols).* These kinematic results are obtained from a consideration of the asymptotic particle trajectories, so they are valid for any type of central scattering potential.

Scattering under the influence of a central potential V(r) takes place in a plane. The geometry of the scattering process is depicted in Fig. 1, where the target is initially at rest. This coordinate system is known as the Laboratory coordinate system (Lab system or Lab frame), to distinguish it from the Centre-of-Mass reference frame (COM system or COM frame) introduced for analytical convenience in the following section.

Figure 2.1. Scattering geometry for a binary collision. The projectile of mass M1, and energy E0, approaches a stationary target of mass M2 located on the x-axis. After the interaction, the motion of the projectile is characterised by the scattering angle q and its asymptotic energy E1, while that of the target is characterised by the recoil angle f and its asymptotic energy E2 (= E0 - E1 in an elastic collision). b (the off-axis distance of the projectile trajectory) is the impact parameter.

The most useful equation produced by the kinematical theory is that which relates the collisional energy transfer to the projectile scattering angle in the Lab system:

E1 = E0(M1M2/(M1+M2))2{ cos q  ± [(M2/M1)2 - sin2 q]½ }2 .                                          (2.1)

Equation 2.1 simplifies considerably if q = 90° or 180°. This equation is the basis of ion scattering spectrometry (ISS), which involves the measurement of E1 for fixed E0, M1 and q, and thereby allows the mass M2 of the scatterer to be deduced.

For (M2/M1) ³ 1, only the positive sign before the [(M2/M1)2 - sin2 q]½  term is applicable. Light projectiles are backscattered from heavy targets in small impact parameter collisions, whereas heavy projectiles push light targets ahead of them, and themselves only undergo small deflections.

The following remarks apply to a potential which falls off monotonically with separation. If M1 < M2, the projectile may be scattered over the entire range of q (0-180°). However, for a heavy projectile (M1 ³ M2), the deflection angle q increases as b increases, up to a maximum value given by sin(q) = M2/M1. This trend is illustrated by the plot of trajectories shown in Fig. 2 (for argon scattering by a carbon atom target) which is taken from ref. [].

Figure 2.2. Trajectories followed by 1 keV Ar projectiles (mass 40) scattered from a carbon target (mass 12, initial location shown as black dot). The impact parameters of the various trajectories are separated by 0.05 Å.

The energy of the recoiling target particle is given by the kinematic theory as:

E2 = E0*4M1M2 cos2 f/(M1+M2)2 .                                                                                      (2.2)

The angles q and f are related by a fairly complicated expression (it is easier to relate f to the scattering angle in the COM system). According to equation (2.2), the maximum energy transferred to the target particle in an elastic collision, Emax, is:

Emax/E0 = 4M1M2/(M1+M2)2,                                                                                         (2.3)

which occurs when the impact parameter is zero: a so-called direct impact (or centre-to-centre) collision. Equation 2.3 implies that energy transfers decline as the disparity in masses of the particles increases. For M1 = M2, Emax/E0 = 1, whereas for M1 = 0.1M2 (or M2 = 0.1M1), Emax/E0 = 0.33. In a proton-electron collision, Emax/E0 = 0.002.

Gryzinski gives a very thorough review of the results of the kinematic theory, including the cases involving an inelastic energy loss or a non-stationary target.

Spider uses the equations 2.1 and 2.2 to calculate the results shown in its ‘Scattering Relations’ window.

2. 3. Dynamics of Binary Collision

2.3.1. The Scattering Cross-Section

The dynamical theory of scattering is concerned with the determination of particle trajectories and scattering cross-sections for specific interaction potentials. To simplify the discussion we assume that M1 < M2, and that the potential falls off monotonically with separation.

Consider an experiment in which we measure the fraction of a projectile beam of incident intensity I which is scattered into an angular range q to q+dq by two different target potentials V1(r) and V2(r). In the first case, this scattering will be associated with impact parameters between b1 and b1+db1, and in the second case with impact parameters between b2 and b2+db2. In other words, there is a distinct mapping b ® q of a given impact parameter to a specific scattering angle for each type of potential. In three dimensions, the angular range q to q+dq represents an annular-shaped solid angle dW, as depicted in Fig. 3.

dW

b+db

q+dq

Figure 2.3. Depiction of a scattering process in three dimensions, showing how projectiles incident within an annulus defined by impact parameters in the range b to b +db are scattered into a solid angle dW.

If the incident particle flux impinging on the target has a uniform cross-section, then the number of incident particles in a small impact parameter range  would be proportional to the area of an annulus of radius b and width b+db, i.e 2pbdb. Due to particle conservation, the number of particles N(W) scattered into the specified solid angle dW (representing the area of a detector) would also be proportional to the area of this contributing annulus of impact parameters. The scattering cross-section s(W), which can be expressed as a function of  q (as well as E0,M1 and M2), is the factor which relates the flux of incident particles to the number of scattered particles detected at a particular deflection:

N(W) = Is(W)dW                                                                                                                  (2.4)

The concept of a scattering cross-section is useful when comparing the behaviour of a similar process in two different scattering systems: for example: ‘the cross-section for 90° scattering is higher for Cu than for Al’ (meaning, more scattered particles are detected for Cu at this angle).

If the potential is non-monotonic, or if M1 > M2, then scattering by an angle q may be associated with more than one value of b (see Fig.2, for example). Effects of the first kind are important in determining the cross-sections of low-energy scattering processes, where they give rise to the rainbow scattering effect.

The concept of a scattering cross-section is closely related to that of a collision radius (or alternatively, the collision diameter). Some physical processes (e.g. certain kinds of electron emission) can only be initiated if the collision impact parameter is below some critical threshold, bmin, known as the collision radius. The cross-section for such a process is equal to the area of the circle defined by a radius bmin. The cross-section concept is often replaced by the idea of a yield in solid-state scattering processes (e.g. the sputter yield, the secondary electron yield and so on): the yield is a measure of the average detection/emission probability for a signature particle associated with the process, referenced to the flux of stimulating particles (electrons per incident ion, etc.).

2.3.2. The Coulomb Potential

The Coulomb potential is described by an inverse square force law (1/r2), which gives rise to a potential energy term of the form:

U(r) = Z1eF = Z1Z2e2/(4pe0r),                                                                                        (2.5)

where F is the electrostatic potential at a distance r from a point charge Z2e, and Z1e is the charge of the interacting projectile. (In a nuclear/atomic context, Z1 and Z2 represent atomic numbers, while e is the charge of the proton). e0 is the vacuum permittivity constant (8.854x10-12 F m-1). The functional form of F (Z2e/(4pe0r)) is derived from elementary electrostatic theory.

The equation of energy conservation for the Coulomb scattering problem in the Lab frame is:

½ M1v12 + ½ M2v22 + Z1Z2e2/(4pe0|r1 - r2|) = E0,                                                           (2.6)

where vi (i = 1,2) is the velocity of the ith particle (= |ivx +jvy|).

The goal of the analysis is to predict the scattering angle as a function of the impact parameter, b. The method of solution involves, as a first step, expressing the problem in a new system of coordinates (R, r12) which are linear combinations of the coordinates (r1, r2) in the Lab frame; the first of these represents the position of the system centre-of-mass (COM):

R = (M1r1 + M2r2)/(M1+M2),                                                                                            (2.7)

while the relative particle position (r12) is given by:

r12 = r1 - r2                                                                                                                           (2.8)

In terms of these variables, equation (2.6) can be expressed as:

½ MV2 + ½ mv122 + Z1Z2e2/(4pe0r12)  = E0,                                                                                                                  (2.9)

where V = dR/dt, v12 = dr12/dt, M(the total mass) = M1+M2, and m (the reduced mass) = M1M2/(M1+M2). Equation 2.9 implies the following equations of motion:

Md2R/dt2 = 0                                                                                                                     (2.10)

m.d2r12/dt2 = -¶U(r12)/¶r12 = Z1Z2e2r12/(4pe0r123)                                                            (2.11)

According to equation 2.10, the COM will be in uniform motion throughout the collision, giving rise (equation 2.9) to a constant associated kinetic energy of ½ MV2. At the start of the collision, the interaction term is negligible (r12 is large), and the target has zero velocity (v(0)12 = 0), so equations 2.6 and 2.9 can be expressed as:

½ MV(0)2 + ½ mv(0)122  = Ecom + E12 = ½ M1v(0)12  = E0,                                                  (2.12)

where Ecom is the constant kinetic energy associated with the COM motion; E12 (the relative energy, or energy in the COM frame), which also constant, is the energy available for the pseudo 1-body scattering problem expressed by equations 2.9. The important point to appreciate is that the distance of closest approach (and other features of the collision) depends on the relative energy, E12, rather than the Lab incident energy, E0. By using the relation:

½ MV(0)2 + ½ mv(0)122  = ½ M1v(0)12,                                                                                  (2.13)

it is not difficult to show that:

E12 = M2/(M1+M2)E0.                                                                                                        (2.14)

Evidently, E12 approaches E0 as the target mass increases relative to the projectile mass. Equation 2.9 can be expressed as:

½ mv122 + Z1Z2e2/(4pe0r12)  = E12 = M2/(M1+M2)E0.                                                        (2.15)

Example. A 1 MeV proton is projected at a stationary alpha particle (He nucleus). What is the distance of closest approach (rmin) in a direct impact collision? Would it be the same if the alpha particle were projected towards the proton with the same initial energy?

Solution. The distance of closest approach is reached when all of the relative kinetic energy E12 is converted to potential energy. Hence, the first term in equation 2.15 is zero, giving: Z1Z2e2/(4pe0r12)   = M2/(M1+M2)E0. Using Z1 = 1, Z2 = 4, e = 1.6x10-19 C, M2/(M1+M2) = 0.8, e0 = 8.85x10-12 F m-1, E0 = 106e J, we obtain r12 = 7.2x10-15 m. For the case of an alpha particle projectile, the M2/(M1+M2) term is now 0.2, and the distance of closest approach is accordingly four times larger (2.8x10- 14 m).

Eqn. (2.11) is formally equivalent to the equation of motion of a particle of mass m moving under the influence of a Coulomb potential originating from a fixed centre. The equation of motion 2.11 can be solved analytically. The Cartesian coordinate system is converted to a circular system: (x12, y12) ® (r12, q). Standard texts show that the scattering angle in this coordinate system (the COM system) can be expressed as: Error: Reference source not found

¥

q = p - 2ò bdr12/r12Ö[(1-U(r12)/E12)r122 - b2] .                                                                    (2.16)

rmin

where U(r12) = Z1Z2e2/(4pe0r12). This integral can be evaluated for the Coulomb potential, as well as a few others such as the inverse square potential (the latter is quite useful for testing simulation programs because it represents a screened Coulomb potential).

For the Coulomb potential, V(r12) = A/r12, equation 2.16 evaluates as:

q* = p - 2arctan(2bE12/A),                                                                                                 (2.17)

where the asterisk (*) reminds us that the angle in question refers to the COM coordinate system. The corresponding result for the inverse square potential, V(r12) = A/r122 is:Error: Reference source not found

q* = p[1 - 1/Ö (1+A/b 2E12)]                                                                                              (2.18)

The COM result, equation 2.17, can be expressed in the Lab frame by means of transformations shown in the standard texts on mechanics, and in a similar fashion, scattering cross-sections for the laboratory system can be derived. These are not particularly interesting for the present discussion, so they are passed over here.

However, it is important to emphasise that knowledge of the Lab scattering angle is not enough to specify the particles’ trajectories in the Lab frame, although it does specify their asymptotic Lab directions of motion. To know the trajectory, one would have to solve for the variation of r12 with time, and express this motion in the coordinates of the Lab frame. This is quite an involved problem, which also requires the evaluation of the so-called time integral, which gives the interval t  between the initial and final limits of the motion:

t = ò dr/{2/m[E-U(r12)] -M2/m2r122}½                                                                               (2.19)

For the Coulomb potential, this integral diverges as the initial radial separation increases, which means that the ideal case of a projectile ‘approaching from infinity’ cannot be evaluated. Computer simulations of atomic collisions based on the binary collision approximation (BCA) model do make use of the time integral (in conjunction with a screened Coulomb interaction). The reader is referred to the research literature for further discussion of this topic. In simulations of atomic scattering in solids, it is important to be able to track the actual motion of the projectile (not just its asymptotes) because of the need to calculate the impact parameter for a possible subsequent collision with another target atom.

2.3.4. Other Potentials

The integral in equation 2.16 can only be evaluated analytically for a few potentials. A number of approximate analytical treatments have been developed in response to this. The small-angle (or impulse) approximation introduces a number of assumptions which simplify the scattering integral, but it is not applicable to hard (small impact parameter) collisions.Error: Reference source not found Another approach is to develop universal formulae based on regression fits to parametric models (so-called ‘magic’ formulae). Approximate techniques of this kind have an important role to play in simulation models based on the binary collision approximation (BCA), where a multiple scattering process is represented as a sequence of binary collisions. The BCA model is the basis of some well-known computer simulation programs (TRIM, MARLOWE). The BCA model will not be discussed further in this article: see Eckstein’s book for a compact review of the model,Error: Reference source not found or the article by Robinson for a comprehensive review.Error: Reference source not found

The ‘composite’ interaction potentials used in classical dynamics simulations of projectile-solid collisions are described in chapter 4.

2.3.4. Many-Body Systems

The equations of motion of many-body (N > 2) systems are not in general amenable to solution by analytic methods. To the author’s knowledge, the most complex scattering system which has an analytic solution is the problem of projectile scattering by two fixed scattering centres under a 1/r potential. More interesting (from the viewpoint of the surface analyst) is a system of two fixed centres governed by screened Coulomb potentials: no analytic solution is possible, but the theoretical analysis of this system by approximate methods remains a subject of current surface science research.

In the normal conception of a many-particle system, the scattering centres (target particles) would undergo displacement as a result of interaction with each other as well as with the projectile. Generally in this article it will be assumed that all interactions can be expressed using pairwise potentials. An N-body system accordingly gives rise to ½N(N-1) pairwise interactions which govern motion in the system. The motion of the ith particle in a many-particle system is then determined by the resultant of all of the forces acting upon it, which can be expressed in the following vector equation (mai = Fi):

Mid2ri/dt2 = -S  (¶U(|ri - rn|)/¶ri)   [ = -S  ÑiU(|ri - rn|) ]                                     (2.20)

where the summation runs over the index n (n = 1...N, omitting the value i).

Here is a concrete example of the evaluation of equation 2.20, for the case when particle 1 is interacting via Coulomb forces with two others (i = 1, n = 2,3):*

M1d2x1/dt2 = Z1e2/(4pe0).{ Z2(x1-x2)/r123 + Z3(x1-x3)/r133 }                                        (2.21a)

M1d2y1/dt2 = Z1e2/(4pe0).{ Z2(y1-y2)/r123 + Z3(y1-y3)/r133 }                                       (2.21b)

M1d2z1/dt2 = Z1e2/(4pe0).{ Z2(z1-z2)/r123 + Z3(z1-z3)/r133 }                                        (2.21c)

Unlike the case of the binary collision, there is no advantage in transforming Newton’s equations to a non-Cartesian or COM coordinate system for many-particle systems. Equations analogous to 2.1 are the starting point for computer-based classical dynamics calculations.

3. Computer Implementation

3.1. Introduction

This chapter reviews some miscellaneous ideas about the development of programs for classical dynamics (CD) simulations. The chapter is intended to provide a few practical tips for research students who are commencing investigations in this area. Other readers may omit this chapter entirely.

It is worth remarking that the development of PC based simulation programs is only possible because of the rapid increase in CPU speeds which have achieved seen since 1990 or so. Robinson mentions that a CDC 7600 mainframe in ~1980 required about 1 s for each integration timestep involving a ~1000 atom system, which is similar to the performance of Snook 2.1 running on a Pentium PC in 1997.Error: Reference source not found However, it must be emphasised that a fast CPU cannot compensate for a poorly implemented simulation algorithm, which can increase computation time by several orders of magnitude.

A first attempt at writing a simulation program will usually produce an inefficient result; but if it runs correctly, the programmer should be encouraged, rather than discouraged, by his creation.

3.2. Computer languages and tools

Traditionally, CD programs have mostly been developed in Fortran, or to a lesser extent in C. (Examples of such programs can be found at many Internet web sites.) There are good reasons (e.g. portability) for continuing to work in these languages when command-line mainframe programs are being developed. For a PC based application, these languages can also be used, but they suffer the drawback that they are not particularly well-supported by the mainstream compiler manufacturers, and so lack the sophisticated ‘rapid application development’ (RAD) features for visual development associated with C++ and Pascal products.

Visual programming and user interface development is not a normal feature of most computer courses for scientists. It is probably wise to write early simulation routines as simple command-line Dos applications. Success in this task will increase motivation to tackle the visual programming hurdle. This is particularly important if you plan to pass on the fruits of your labours to others. Be warned that today’s PC users may simply ignore a scientific program that lacks a menu-driven user interface! Fortunately, with a compiler product like Delphi or C++ Builder it is almost child’s play to develop a minimal interface.

C++ and (Borland-) Pascal are object-oriented languages which take most of the labour out of designing windows, menus, dialog boxes and other visual objects. Although it is not difficult to link together object files built from different languages, most programmers  would presumably prefer to develop and debug their programs from within a single, integrated environment. (For C, this can of course be achieved with a C++ compiler.)

The big names in compiler technology today are Watcom, Microsoft and Borland. Modern compiler products feature a high degree of integration between the code editor, the compiler, and various supporting tools (including, hopefully, a debugger and profiler). For scientific applications, it is very important to have (a) the capability of inserting assembly language statements into the body of high-level language code; (b) a debugger which allows you to view the registers of the numeric coprocessor. (You may not use assembly language now, but you will eventually.) You may have to buy a tool for (b) separately (if you have Delphi 2 or 3, for example, you will need Turbo Debugger 32).

If you plan to distribute your program, you may need to develop a context-sensitive, online Help system for it. For Windows environments, the use of a help-authoring tool is recommended for this task. You can find shareware products in the \winhelp directories of Simtelnet’s win95 and win31 collections.*

The final class of tools I want to mention comes under the description of memory checkers for instance HeapAgent (http://www.microquill.com) and BoundsChecker (http://www.numega.com). These are fantastically expensive tools, but you can download and use them for a limited period to run over your final program, which they will check for various kinds of memory and resource allocation errors.

3.3. Performance issues

It is meaningless to argue along lines such as ‘Fortran is faster than C++’: the relative performances of two programs depend on how the compiler manufacturers have chosen to implement floating point operations at the assembly language level. The relative performances can also vary according to the hardware, operating environment and compilation options. Borland Pascal 7, for example, used a very slow implementation of the exp() function, but the same function in its 32 bit successor (Delphi 2) is as tight and fast as the coprocessor architecture allows. Essentially, performance has to be determined by empirical investigation.

A code profiler (such as Turbo Profiler) may be shipped with your compiler, though I know of none yet for the Windows 95 environment. This tool allows you to determine the ‘hot-spots’ or ‘time-eaters’ of your program, and thereby develop strategies for performance optimisation. For example, a CD simulation program spends most of its time either computing the Morse forces, or building the neighbour lists. You can use a profiler to identify the critical statements which you need to (a) rewrite in a high level language or in assembly language, or (b) replace by a faster algorithm (the preferred method).

Program precision is likewise dependent on low-level language and hardware details, rather than the choice of high-level language. The double-precision IEEE floating point types used by Fortran, C, C++ and Pascal are identical. However, hardware (numeric processor) performance varies from manufacturer to manufacturer.

3.4. Programming tips

3.4.1. Programming style

From the outset, you should write your code in the expectation that it will have to be modified many times. This means choosing meaningful variable names, and documenting the logic of obscure algorithms (Fortran and C users take note!). As far as possible, you should compile and run the program as soon as each new function/subroutine is completed: this may involve writing some temporary skeleton code which calls the new function for testing purposes. (The ability of PC compilers to integrate editing/compilation/debugging operations is a major advantage over mainframe development environments.) In this respect, the fast compilation time of Pascal is a major advantage over C++.

3.4.2.  Operating systems

Since PC operating systems and compilers change so fast nowadays, it is essential to separate the code for your user interface (‘front end’) from that for your simulation routines (‘back end’ or ‘engine’). For example, the simulation engine used by Snook consists of one big function (subroutine) contained in a module of its own which is called by the main program:

...

Simulate(TrgFile, PrjFile,...);

...

The same function can be used with minor modifications by a Dos program or by a Windows 95 program. Once the simulation is running, the main difference between these two platforms is in the way user inputs (e.g. Ctrl-Break) and screen outputs (e.g. messages) are handled. Instead of executing your chosen input/output method directly, you should create a procedure for the purpose:

...

Msg := ‘Output information’;

WriteScreen(Msg)

...

Now when you change OS platforms, only the WriteScreen() routine has to be changed.

3.4.3. User input

Input data should be validated as much as possible at the time of entry (in the Simulation Kit, most data validation is handled by Spider). Modern visual development environments provide dialog box ‘objects’ which can automate a wide variety of data validation tasks (e.g. validating the range and format of floating point and integer numbers).

Avoid ‘hard-coding’ data into your simulation routines as much as possible. It is tempting to do this when you are the sole user of the program, because you can always change the data and recompile the program as you wish.  However, in the author’s experience, this procedure tends to introduce errors. If you find yourself changing hard coded constants, it is better to provide some method of inputting them into the program from an external source.

Ideally, input should be in the form of data stored in files, rather than data entered via the keyboard. File-based data mean less work for the user, and are self-archiving. However, for small programs (such as the cone utility) it may be too pedantic to insist on file-based inputs.

3.4.4. Dynamic memory issues

You can write simulation programs without using pointers and dynamic memory allocation. But eventually you may wish to do so. In a Dos/Win 3 environment, one quickly exhausts the 64 k array size limitation, which applies regardless of your machine’s physical RAM. Under Windows 95/NT, more memory is available, but it has to be shared with other programs.

In general, dynamic memory allocation and deallocation is an area of programming which always requires special attention:

(a) Failure to deallocate memory will cause memory ‘leaks’. The easiest way to check for these is to continuously monitor memory usage. You can see both Snook 1.2 and 2.1 offer options of this kind, in the form of visual memory meters. These are used by the author for debugging purposes. After running a simulation, the memory allocation should return to its original value. For a variety of reasons, such as pointer management overheads, this does not usually occur precisely. However, apparent leaks due to legitimate overheads will saturate after the routine has been run 2 or 3 times, unlike true leaks which will continue to deplete the memory pool ad infinitum.

(b) Failure to allocate sufficient memory (e.g. for arrays whose length is set at run-time) is another source of dangerous bugs, particularly the well-known ‘out-by-one’ error that plagues C programmers. Such bugs (memory overwrites) will not necessarily be indicated (by a general protection fault) even in a protected mode environment, so it is good practice to examine the contents of arrays (or at least, their first and last elements) in the ‘Watch’ window of a debugger as you step through the program. Once things are running, it is a good idea to modify the program so that that you are deliberately under-allocating memory, in order to see what behaviour this produces.

(c) Most non-trivial program errors are associated with illegal pointer operations. To keep these to a minimum, it is a good idea (a) to assign pointers to nil (or null) both on program start-up, and after deallocating dynamic memory; (b) to allocate or deallocate memory for a given pointer in only one place in the program.

3.4.5. Compiler bugs

Know your compiler well: most libraries and compilers released by compiler vendors contain dozens of bugs. (Lists of these can be found on Internet sites and in Usenet groups comp.lang.xxx devoted to that compiler.) For this reason, you need to ascertain that your compiler package contains the source code for all libraries, so that you can debug them if necessary.

3.4.6. Data structures and algorithms

For classical dynamics simulations, the main data structures you will use are the 1- and 2-dimensional arrays. 1-dimensional arrays are used to hold vector components (x[n], y[n], z[n]), while 2-dimensional arrays are used for holding neighbour lists and possibly force components (depending on the integration algorithm you use). Do not use container classes (objects) to store vector quantities, because access to their elements is needlessly slow.

In Dos, it is useful to be able to size 2-dimensional arrays (a) beyond the 64 k barrier, and (b) to do this at runtime.  A data structure which can do this is declared and implemented as follows.

type

float = double;

PFloatArray = ^TFloatArray;

TFloatArray = array[1..8000] of float;

PFloatMatrix = ^TFloatMatrix;

TFloatMatrix = array[1..8000] of PFloatArray;

var

F: PFloatMatrix;

n, ncells, mcells: integer;

begin

{ allocate memory for an array of size ncells*mcells}

{ ncells, mcells must each be < 8000 in this example }

ncells := 1000;

mcells := 100;

{ total memory allocation is ncells*mcells*4 bytes }

GetMem(F,ncells*SizeOf(PFloatMatrix));

for n := 1 to ncells do GetMem(F^[n],mcells*SizeOf(PFloatArray));

{ initialise first cell }

F^[1]^[1] := 0.0;

{ initialise last cell }

F^[ncells]^[mcells] := 0.0;

{ free memory }

for n := 1 to ncells do FreeMem(F^[n],mcells*SizeOf(PFloatArray));

FreeMem(F,ncells*SizeOf(PFloatMatrix));

end;

CD simulations do not usually require an in-depth knowledge of computer science algorithms beyond what can be gleaned from numerical analysis texts like Numerical Recipes. A possible exception is range-searching algorithms (for improving the efficiency of neighbour list construction).,Error: Reference source not found

3.5. Program structure

In this section I want to highlight some fragments of code extracted from Snook for two purposes. First, the code fragments will shed light on the internal structure of the simulation routine. Also, they may be of interest for those who plan to develop their own routines. I have simplified the appearance of the code by replacing dereferenced pointers by static variables (in practice, memory for all array variables is allocated dynamically), and by removing any details of minor interest.

It should be appreciated that the bulk of the code in Snook’s simulation routine (i.e. excluding the user interface and event handling system) is devoted to initialisation processes, namely (a) memory allocation, (b) data input/validation, and (c) assignments to variables. The initialisations performed in (b) and (c) can be divided into two types: those that must be initialised once only (such as the potential parameters), and those that have to be initialised for every new run, i.e. every new projectile trajectory (for example, the target atom coordinates).

Although the bulk of the programming effort is devoted to the initialisation routines, the bulk of execution time is spent in computing the forces and in updating the neighbour lists. Most optimisation efforts will be focussed on these routines, and it is these sections which have to be constructed most carefully.

3.5.1. The integration loop

The code fragment shown below implements the ‘velocity form’ of the Verlet integration algorithm:

rn+1 = rn + vnDt + ½ FnDt2/m                                                                                (3.1)

vn+1 = vn + ½ (Fn+1 + Fn)Dt/m                                                                             (3.2)

On entry to the procedure, some useful composite constants are prepared, and the arrays, which on exit hold the force components (from the current and preceding timesteps), are initialised. The forces are summed by calling the GetForce() procedure, and finally the particle velocities are updated.

Procedure Get_rv_Verlet(NoOfAtoms:longint;ti:float);

var

i,j,k: longint;

Fx, Fy, Fz: float; // used to hold forces

ti2div2,tidiv2,CutOff2: float; // composites

kr,kv: float; // Composites

ArraySize: longint;

begin                     // Get_RV_Verlet

ti2div2 := 0.5*sqr(ti); // ti is the timestep

tidiv2 := 0.5*ti;

CutOff2 := sqr(CutOffDistance);

for i := 0 to NoOfAtoms do // FNew gets value of Force from prev. step

begin

FNewX[i] := FOldX[i];

FNewY[i] := FOldY[i];

FNewZ[i] := FOldZ[i];

end;

ArraySize := (NoOfAtoms+1)*SizeOf(float);

FillChar(FOldX,ArraySize,0);  // zero out

FillChar(FOldY,ArraySize,0);

FillChar(FOldZ,ArraySize,0);

// Calculate predictor for r

for i := 0 to NoOfAtoms do    // Calculate drift during dt

begin

kr := ti2div2/mass[i]; // = ti^2/2m

x[i] := x[i] + vx[i]*ti + FNewX[i]*kr;

y[i] := y[i] + vy[i]*ti + FNewY[i]*kr;

z[i] := z[i] + vz[i]*ti + FNewZ[i]*kr;

end;

// Evaluate forces at new r

for i := 0 to NoOfAtoms do

begin

k := 0;

j := partner[i][k];  // j is partner #k of i

while  (j <> EndOfData) do   // EndOfData is end of partner list

begin   // j is always > i in the partner array

if (sqr(y[i]-y[j])+sqr(x[i]-x[j])+sqr(z[i]-z[j]) <= CutOff2) then

begin

Get_Force           // at r[n+1]

(Fx,Fy,Fz, // <-- outputs these force components

x[i],x[j],  // interaction of #i with its k_th partner

y[i],y[j],  // which is particle #j

z[i],z[j],

i,j);

FOldX[i] := FOldX[i] + Fx;

FOldY[i] := FOldY[i] + Fy;

FOldZ[i] := FOldZ[i] + Fz;

FOldX[j] := FOldX[j] - Fx;

FOldY[j] := FOldY[j] - Fy;

FOldZ[j] := FOldZ[j] - Fz;

end;

inc(k);

j := partner[i][k] // Break when j = EndOfData

end;     // while j

end; // i

for i := 0 to NoOfAtoms  do  // update velocity

begin

kv := tidiv2/mass[i];

vx[i] := vx[i] + (FNewX[i] + FOldX[i])*kv;

vy[i] := vy[i] + (FNewY[i] + FOldY[i])*kv;

vz[i] := vz[i] + (FNewZ[i] + FOldZ[i])*kv;

end;     // for i

end; // Get_rv_Verlet

Each atom, #N, has a list of potential neighbours or collision partners which is stored in the partner array (the ‘neighbour lists’). Thus, the partners of atom #N are stored as integer indexes as the array elements partner[N][0], partner[N][1], partner[N][2] and so on, until a negative index signals the end of the list for atom #N. This arrangement means that time is not wasted in checking the location of all atoms in the lattice. The neighbour lists are updated periodically. To speed things further, the only atoms included in the neighbour list of an atom of index #N are those whose indices are greater than N. This means, for example, that F31 (the force acting on particle #3 due to interaction with #1) is computed at the time that F13 is computed, exploiting the fact that F13 = -F31.

The GetForce routine returns the force components calculated from the particle vectors. Indices (i, j) identifying the particles are also passed to the routine. Based on these indices, the routine can decide whether the force should be calculated from the projectile-target or target-target potential parameters respectively.

3.5.2. The timestepping loop

The main loop of the program has a very simple structure. As shown in the simplified fragment below, this loop repeatedly calls the integration algorithm discussed in the preceding section. The loop also has responsibility for initiating rebuilds of the neighbour lists (see next section), and for updating the timer (time elapsed), the timestep and the interaction sheath (‘test range’) respectively.

// Perform various initialisations of variables (deleted)

// Start main loop

Repeat

if (NeighbourListTimer = UpdateTime) then

begin

CorrectTestRange; // Shld be called before GetNeighbours

GetNeighbours(NoOfAtoms);    // build neighbour list

NeighbourListTimer := 1;     // reset counter

if UserAborted then break;   // break from Repeat..until

end

else inc(NeighbourListTimer);     // increase counter

Get_rv_Verlet(Lattice,NoOfAtoms,ti)

TimeElapsed := TimeElapsed + ti;  // update elapsed time counter

inc(TimeStepsExecuted);           // update timesteps counter

CorrectTimestep;                  // Update ti, V0

Until TerminationConditionsMet;       // time & energy conditions

3.5.3. The neighbour lists

The routine below shows how neighbour lists are built up in Snook by the ‘brute force’ method. For a given atom i, the algorithm prepares a list of those atoms j,k,l... which lie within a certain range. Some work is saved by using a projective technique to filter out particles which are found to be out of range along a particular axis. However, for large lattices, this is not the fastest way to build up the lists. Snook offers a second option for building the neighbour lists (the ‘box method’), which will not be presented here, as its algorithmic details are rather tedious.

Procedure GetNeighbours(NoOfAtoms:longint);

var

i,j,k: integer;

range:float;

begin

// User key-presses/mouse clicks are processed here (not shown)

range := sqrt(TestRange2);  // range = width of interaction sheath

for i := 0 to NoOfAtoms do  // i must run over ALL particles

begin

k := 0;

Partner[i][0] := EndOfData;   // Partner = i,j,k,..EndOfData)

for j := i+1 to NoOfAtoms do    // Ignore j <= i

if (abs(x[i]-x[j]) > range) // This condition filters out

or (abs(y[i]-y[j]) > range) // the majority of atoms in the lattice

or (abs(z[i]-z[j]) > range) then  // do nothing

else if (sqr(x[i]-x[j])+sqr(y[i]-y[j])+sqr(z[i]-z[j]) < TestRange2)then

begin

Partner[i][k] := j;

inc(k);

if (k = NoOfPartners) then  // Stop! (not shown)

end;           // for j

Partner[i][k] := EndOfData;

end;       // for

end;           // GetNeighbours

4. The Physical Model

4.1 Introduction

This chapter will review the physical model and assumptions underlying the simulations performed by the Simulation Kit program Snook. All simulation models are founded upon a multitude of assumptions. To use and interpret the results from a simulation, one must understand the limitations of the simulation as well as its capabilities.

It is extremely difficult to prove the physical (as opposed to computational) correctness of any particular simulation model. The observation of individual projectile trajectories or the ensuing collision cascades is not experimentally feasible, so only average quantities derived from simulations can be checked empirically. Effectively, the information generated by simulations for a 6N-dimension phase space is mapped onto a 6-dimension space for purposes of experimental comparison. Any simulation which produces an experimentally-corroborated mean distribution of particles in phase space will thus appear to correctly represent the physical system, even if it is correct “by accident.”* Furthermore, many experimental scattering techniques sample a phase space of limited dimensionality: e.g. ion scattering spectroscopy samples only px and pz (and possibly py), and therefore only tests a restricted number of the simulated phase space averages.

In the following brief review, I have tried to address the issues which I think will be most important for an understanding of the design of the Simulation Kit. I hope that readers will point out omissions to me. I have not attempted to discuss competing classical dynamics simulation models (e.g. those incorporating embedded-atom or 3-body potentials).

4.2. Initial conditions

4.2.1. Lattice structure

The lattice structure used by Snook is derived from the Target file (*.TRG) which is a listing of coordinates and other information about the particles in the target. The xy plane of the coordinate system is parallel to the surface, while the z-axis corresponds to the surface normal. The negative z-direction corresponds to movement into the lattice. The projectile approaches the surface in the xz plane if the azimuthal angle is set to zero (in the Run file), starting from the +x direction, and moves in the negative x-direction towards the location of the anchor atom (see below).

If thermal displacements are selected (in the Model file), then at run-time Snook will add appropriate (random, Gaussian, Debeye-Waller) thermal displacements, including any surface effects specified by the user. In this case, the user also has an option (under Snook’s Options|Simulation menu) to add an average kinetic energy of 3/2kT to the lattice atoms (which is derived from a Maxwellian distribution of velocities). If this option is not selected, the lattice atoms are stationary at the start of the simulation (the normal case for keV scattering). Chapter 8 discusses the implementation of thermal vibration effects in detail.

4.2.2. Projectile coordinates

The starting position of the projectile is referenced to the coordinates of an anchor atom in the target (see fig. 4.1): these are the coordinates listed in the first line of the Target file (i.e. the coordinates prior to application of thermal displacements). For small impact parameters, the anchor atom is the projectile’s first collision partner. By default, in all lattices generated by Spider the anchor atom is located at the origin (0,0,0) of the coordinate system.* The vertical projectile position relative to the anchor atom is determined by the z0 parameter (normally 3 Å) in the Impact file. The (x, y) coordinates of the projectile are determined (a) by the impact parameters (along the x- and y-directions) for the current run (trajectory), which are listed in the Impact file; (b) by the projectile altitudinal and azimuthal angles of incidence.‡ The angles in (b) also determine the relative magnitudes of the projectile starting velocity components.

z0

Figure 4.1. Starting geometry for a simulation run. The uppermost particle (red) is the projectile, which is located at a vertical distance z0 above the anchor atom (green, third from the left) in the target lattice. The lateral displacement of the projectile relative to the anchor atom is determined by (a) the impact parameter for that run, (b) the projectile angles of incidence. See the Spider on-line Help for more information on this subject.

The arrangement just described has the advantage that it allows the same set of input files to be used for similar scattering systems with different altitudinal directions of projectile incidence (with the sole difference that the particular  altitudinal angle involved has to be specified in the Run file).

4.2.3. Particle masses

The isotopic distributions of the target atoms are not known by the program (Spider) which generates the Target file (the target particles are assigned the user-specified average mass), but it is possible in principle to modify the atomic mass data in the Target file by manual editing of each entry. The projectile mass is likewise fixed at the value stipulated in the Projectile file.

4.2.4. Initial conditions data

If you want to examine the state variables (position, momentum and others) which were actually applied at the start of the simulation, there is an option in Snook which causes them to be dumped to a file inivars.snk, from which they can be regenerated by Winnow’s Process|Convert command.

4.2.5. Reduced impact zone

A simulation project typically consists of a large number (102 - 105) of simulations of independent projectile trajectories, often referred to as ‘runs’ in the Simulation Kit documentation. The trajectories are directed towards a representative region of the surface of the target lattice which the author calls the reduced impact zone. The reduced impact zone is an area of the periodic surface sized in such a way that every possible incident trajectory on the surface can be mapped to a symmetrically equivalent trajectory directed into that zone. The dimensions of the reduced impact zone depend on the surface orientation, and the symmetry inherent in the collision problem (i.e.  the direction of projectile incidence). Fig. 4.2 illustrates the reduced impact zone for a FCC (111) crystal surface. The reduced impact zone is always smallest at normal projectile incidence.

Figure 4.2. Projection of the “representative region” or “reduced impact zone” for projectiles normally incident onto a FCC (111) surface. The atoms at the corners of the zone lie in 3 different layers of the target. (These data are taken from the TRG and IMP files found in the \examples\ne-ag111 directory of the Simulation Kit installation.)

The Impact file of a Snook simulation project contains the impact coordinates which collectively define the dimensions of the reduced impact zone. The Spider online Help includes an extensive discussion on the topic of reduced impact zones, and includes templates for the most common surfaces.

For an ideal lattice, there is no error introduced by using a reduced impact zone which is, for example twice or four times as large as the true zone. However, this can be inefficient in computational terms, as it may introduce redundancies in the output data.*  The problem of collision containment also encourages the use of a restricted impact zone.

The introduction of randomly-applied thermal vibration effects has two consequences in connection with the impact zone; (a) each target configuration is unique when vibrational effects are included in the simulation, so every simulation will yield different results; (b) the thermal displacements blur the notion of symmetrically  equivalent trajectories (indeed, the same projectile starting coordinates will generally give rise to different collision cascades, because of the different displacements of the target atoms).

It is sometimes convenient to use an impact zone that is larger (by an integral factor) than the reduced impact zone. A typical example occurs during the simulation of processes as a function of projectile altitudinal angle, where one may prefer (for the sake of simplicity) to use the same impact zone (i.e. Impact file) at normal incidence as was used for the simulations at oblique angles of incidence. (The preceding paragraph and its footnote explain why even this case will be free of redundancies in simulations that include vibrational effects.)

4.3. Interaction potentials

4.3.1. Interaction Model

Atomic interactions modelled in the Simulation Kit are based on analytic pair-potential functions as specified in the Model file.

The Simulation Kit user is required to select the type of potential employed by the simulations. There are two broad choices: (a) screened Coulombic potential; (b) composite potential. Potential (a) always applies to the projectile-target interaction. The user may select either (a) or (b) for target-target interactions. In both cases the potential is cut-off at a finite distance specified by the user (normally between the 1st and 2nd nearest neighbours, or between the 2nd and 3rd nearest neighbours).

The Morse and Screened Coulombic potentials, although fairly well established, are not the only potentials being used for contemporary simulation research, nor are they probably the most realistic. Their main advantage lies in their suitability for a “universal” approach to simulation, such as that adopted by the Simulation Kit.

The composite potential used for target-target interactions consists of a screened Coulombic potential at small internuclear separations, a Morse potential at large internuclear separations, and a cubic spline function which connects the two in the intermediate region. The spline function is computed automatically, based on the user’s specification of the spline region limits.

Fig. 4.3 shows the attractive part of the default composite Cu-Cu potential generated by Spider’s Model option. The corresponding screened Coulombic potential for Cu-Cu (not shown) decreases far less slowly towards zero in the region shown, and lacks any attractive potential well.

Figure 4.3. Composite potential, V(R), for Cu-Cu interaction. In the region shown, the potential  consists of a cubic spline function (1.5-2.1 Å) joined to a Morse potential (R > 2.1 Å).

4.3.2. Screened Coulombic potential

The short range potential used by Snook is classified as a screened Coulombic potential. There are 3 variants in common use, which are named after their developers: the (Ziegler-Biersack-Littmark) ZBL potential, the Moliere-Firsov potential and the Moliere-Lindhard potential. The screening length correction is an adjustable parameter which may be used to modify the range of these potentials to improve their accuracy.

The user of the Simulation Kit has to select: (a) one form of screened Coulombic potential to represent the projectile atom-target atom interaction; (b) one form of screened Coulombic potential to represent the target atom-target atom interaction; (c) two 'screening length adjustment' factors which typically fall in the range 0.7-1.0 (1.0 is normal for the ZBL potential). The target-target and projectile-target interactions do not have to be of the same form or have the same screening length correction.

The analytic form of a screened Coulombic potential for interacting atoms of atomic number Z1, Z2 respectively, is as follows:

V(r) = Z1Z2e2.c(r/a)/4pe0r                                                                                                 (4.1)

The screening function c(r/a) has the properties c(0) = 1, c(¥) = 0. It is normally represented in the general form:

c(r/a) = i a[i].exp(-b [i].r/a),                                                                                            (4.2)

where the index i runs from 1-3 (Moliere potentials) or 1-4 (ZBL potential), and the a[i] and b[i] are coefficients (the former satisfying S a[i] = 1); a is the screening length (defined below).

The Moliere potentials and the ZBL potential  define their respective parameters (a[i], b[i] and a)  in various different ways.*

The Moliere potential has two variants, according to the way in which the screening length (a, in A) is chosen:

a = 0.4685/(Z10.5+Z20.5)2/3        (Firsov form)                                                                (4.3a)

a = 0.4685/(Z10.33+Z20.33)0.5      (Lindhard form)                                                          (4.3b)

In practice this distinction is irrelevant since (judging from the literature) every user of this potential seemingly adds his or her own screening length correction. The screening length correction is a 'correction' factor (< 1.0) which is used to scale the screening length parameter in screened Coulombic potentials, particularly the Moliere potential. A typical value for this correction for the Moliere potential would be 0.8, usually chosen by fits to experimental data e.g. impact collision ion scattering spectrometry.

The screening length (a, in A) for the ZBL potential is defined as:

a = 0.4685/(Z10.23 + Z20.23)                                                                                              (4.3c)

For the ZBL potential, you would normally choose a value of 1.0 for the screening length correction, unless you have reason to do otherwise.

You should be wary of changing the screening length correction parameter arbitrarily, as it has enormous effects on the potential and unrealistic values could conceivably undermine the credibility of the simulations.

Which screened Coulombic potential is best? This question cannot be answered with rigour. However, if you are not going to search for an optimum screening length correction, the ZBL potential is probably the easiest choice because it is normally used in 'unadjusted form', i.e. with a screening length correction of 1.0. Nevertheless, in a particular case, the Moliere potential with an optimally adjusted screening length may well be closer to reality than the unadjusted ZBL potential.

The problem is knowing what correction to apply to the Moliere screening length. In early work a factor of 0.8-0.9 was typical, but more recently even values < 0.7 seem not to excite comment: e.g. 0.68 was used for 4 keV Ar-Ni in ref. [].

Such 'correction' factors have an enormous effect on the potential, because they involve exponentiation. In fact, the effect of the corrections may be more significant than some of the terms in the original potential!

The ZBL potential is generally used without any screening length correction, and this fixed form may be regarded as an advantage or a disadvantage, depending on how highly you rate the potential. For the Ar-Cu system at least, this author notes that the ZBL potential fits closely the ab initio potential calculated by Broomfield et al. However, the well-known MD studies of the Ar-Cu system by Harrison's group in the late 1970’s were mostly carried out using the Moliere potential (with a screening length correction of around 0.85).

The conclusion must be that in any serious study of an ion-surface collision one should experiment a little with the potential, to see whether this has any influence on the outcome of the simulation averages.

4.3.3. Morse and spline potential

The Morse potential V(r) is defined by:

V(r) = D{ exp-2a (r-r0) - 2exp-a (r-r0) } .                                                                        (4.4)

The parameters D, a and r0 are derived from fits to bulk solid state properties, and may vary slightly from author to author, depending on the assumptions (especially the range of action) involved. The potential is a minimum when r = r0, at which point V(r0) = -D. The parameter r0 is of the same order, but greater than, the lattice nearest neighbour distance.

The Morse potential, or any simple pair potential, is not particularly well suited for modelling polyatomic processes that are rigorously directional in character, e.g. covalent bonding in a material like diamond or silicon. This does not mean that such solids cannot be modelled via a Morse potential, only that effects which depend on directional bonding cannot be modelled. The validity of the centre-to-centre model of interatomic forces (in the equilibrium solid) can be assessed by consideration of the elastic constants of the solid of interest. If the centre-to-centre model holds, then it can be shown for cubic crystals that the elastic constants C12 and C44 (also known as Cxxyy and Cxyxy) should be identical.Error: Reference source not found Here are C12/C44 ratios for some elemental solids: Fe (1.15); Cu (1.61); Ni (1.12); Ag (2.02); Al (2.18) diamond (0.22); Si (0.8); Ge (0.72).Error: Reference source not found,Error: Reference source not found This comparison indicates that the centre-to-centre force model holds better for Si and Ge than it does for Cu, but breaks down badly in the case of diamond.

If you choose to include a Morse potential, Snook will also calculate the cubic spline function required to join the Morse potential to the screened Coulombic potential. You can preview the composite potential (screened Coulombic + spline + Morse) in Spider. Note: this spline potential is recalculated by Snook at run-time: no explicit information about it is written to the Model file.

If you choose not to use a Morse potential, the simulations will use the screened Coulombic potential up to the range specified by the potential cut-off. This is adequate for ISS simulations, for example, because (a) the interaction time is short; (b) the process being modelled involves short-range interactions. Robinson gives a good discussion on this subject.Error: Reference source not found

The projectile-target potential used by Snook is always the screened Coulombic potential, regardless of whether or not you have selected the Morse potential option.

If the Morse potential option is selected, a cubic spline function will be used to join it to the screened Coulombic potential between the limits you specify in the Model file. The objective is to choose spline limits that give a smooth transition between the two potentials (no unphysical minima or maxima). The limits are chosen by trial and error. The composite potential will always show a smooth fit at the spline function limits. However, the force function will normally have a discontinuity in gradient at the upper and lower limits of the range covered by the spline function. The type of discontinuity to avoid is one which introduces a spurious maximum or minimum at one or both of the spline limits.

If, for some reason, you wish to employ a cubic (spline) potential all the way up to the cut-off distance, you can do so effectively by specifying a tiny separation between the high spline limit and the cut-off distance (e.g. cut-off distance = 3.8 Å, high spline limit = 3.7999 Å).

A restriction you need to be aware of is that the spline limits should both be greater than or equal to 1.0 Å. This restriction relates to the way in which the Morse look-up tables are implemented. The condition is enforced by Spider and checked by Snook at run-time.

See the Spider online Help for further comments about Morse parameters.

4.3.4. Surface binding energy

The surface binding energy would not be needed in a classical dynamics simulation if the pair interaction potential completely described the system dynamics. Many workers have argued for the inclusion of a surface binding energy term (planar surface potential or barrier) to describe the non-local, long-range interaction between an escaping atom and the free electrons of the lattice.Error: Reference source not found This kind of model (which is vaguely reminiscent of the pseudo-potential approach in solid state quantum theory ) views the atom-lattice interaction as a superposition of pair potential effects, plus effects caused by a 3-dimensional “potential well.” Trajectories are only influenced by the continuum potential when particles cross the potential barrier, e.g. in sputtering.

By contrast, Harrison argues that in a classical dynamics simulation the surface binding energy correction is unnecessary, because its influence is implicit in the Morse potential deduced from solid state properties.Error: Reference source not found

It is apparent that theoretical models describing “structureless media” can benefit from the introduction of a surface binding energy term, since there are no point sources of potentials. However, it is far from easy to decide on an appropriate value for the surface binding energy term in a classical dynamics simulation which already incorporates a Morse potential. Using Spider’s Target|Neighbour Potential option in conjunction with a Morse potential that cuts off at 3.8 Å, one can calculate a binding energy due to this potential of 2.4 eV, compared to the bulk cohesive energy, U0,  of 3.48 eV for Cu. There are theoretical grounds for expecting the total surface binding energy to be roughly the same as U0.* Consequently, it is seen that the Morse potential model in this case accounts for most of the expected total, leaving less than 1 eV for the barrier associated with the postulated planar surface potential. This is a back-of-the-envelope calculation, but it illustrates how you might go about estimating the importance of a planar potential term.

If you wish to include a surface binding energy term (ES) in your simulation, you can specify its magnitude in the Model file of your simulation project. In the current implementation of Snook, no corrections to ejected particle trajectories will be made until after the simulation has terminated (i.e. the correction is made to the final conditions of the system).

The surface binding energy correction  implemented by Snook takes the form of subtracting an amount Ö(2ES/m) from the vertical (z) velocity component of all particles, other than the projectile, which are found to be emitted from the lattice (the definition of emitted here is that the vertical (z) particle separations from the surface are greater than the cut-off distance of the potential). The net effect of the correction is to apply a 'refraction' to the trajectories of particles that left the surface. Specify a value of 0.0 for the Surface Binding Energy if you don't want to include a surface energy correction. (This practice is used by the author in all of the examples that accompany the Simulation Kit.)

A more 'rigorous' approach to the implementation of a surface binding energy correction would be to subtract the surface binding energy at the instant when the particle leaves the surface, but this is not done by Snook, because of difficulties of applying the correction in a consistent manner. The procedure used by Snook tacitly assumes that ejected particles move in free space: above-surface interactions cannot be easily accomodated within a planar potential model of this type (fortunately they are rare).

If you choose to include a non-zero surface binding energy, you will have to select a suitable value based on sources external to the Simulation Kit. None of the example projects discussed in this article incorporated surface binding energy effects.

4.3.5. Image potential

A charged particle approaching the surface of a conductor experiences a classical image potential V(r) = e2/16pe0r, where r is the particle-surface separation.Error: Reference source not found If r is expressed in Å, then V(r) = 3.6/r eV. Since most primary projectiles are charged particles, there is clearly some possibility that image potential effects can influence the trajectories of slow projectiles. This would take the form of an acceleration towards the surface along the z-direction, with a corresponding increase in projectile kinetic energy (on the order of a few eV).

The Simulation Kit does not yet implement the handling of image potential effects. This subject is rarely discussed in the literature of scattering simulation, although it may be important in connection with the angular distribution of secondary ions, for example. A complicating factor is the strong likelihood that charged particles will be neutralised at distances of 2-3 Å from the surface, thereby eliminating the image potential.Error: Reference source not found However, if you plan to run simulations involving projectiles of less than ~100 eV energy, you should at least be aware of the possibility of image potential effects, which will tend to increase the projectile velocity in the z-direction.

This topic will be given a better coverage in a future release of this article.

4.4. Integration methods

The classical equations of motion are integrated by Snook using any of several finite difference integration algorithms. The Verlet algorithm, which is used by default, has already been presented in eqns. 3.1 and 3.2 of chapter 3. Another algorithm offered in Snook’s simulation options is the HGE-A algorithm, which is a so-called predictor-corrector method:Error: Reference source not found

Predictors:

rn+1 = rn + vnDt + Fn Dt²/2m                                                                                               (4.5a)

vn+1 = vn + 0.5[Fn+1 + Fn] Dt/2m                                                                                       (4.5b)

Corrector for rn+1:

rn+1 = rn + vnDt + 0.25[Fn+1 + Fn] Dt²/2m                                                                          (4.5c)

The meanings of the symbols are: rn, vn, Fn: position, velocity, total force vectors at nth timestep; Dt: size of the current timestep.

These integration algorithms are uncontroversial. In general, their accuracy is checked with reference to the accuracy of energy conservation, and this is how the user should evaluate them. The accuracy of the integration depends on the timestep specified by the user in the Run file of a simulation project. One integration algorithm offered by Snook, the Beeman algorithm, should be used with a fixed timestep, and is therefore considerably more inefficient than the other choices. In general, the Verlet algorithm should prove adequate for all simulations, but it may be interesting to check one of the others for consistent behaviour from time to time.

The total system energy, as the sum of potential energy (PE) and kinetic energy (KE), can be calculated at any instant from the current particle positions (using the analytic potentials) and velocities. This can be compared with the energy calculated at the start of the simulation, which yields the energy error DE. Snook uses the formula DE/(KE + PE) to report energy conservation, but since the PE is usually negative, this formula tends to overestimate the importance of the error compared with the alternative measure: DE/(KE + |PE|) used by some writers. However, since DE is normally less than 1% the distinction is not particularly important.

The forces appearing in eqns. 4.5 are not necessarily calculated directly from the analytic potential functions. In Snook, Morse forces are calculated from a look-up table (essentially by indexing the inter-particle separation, r, into an array of pre-calculated values, with interpolation if necessary). Other forces (spline, screened Coulombic) are calculated directly from the respective analytic functions. The majority of particles in the system interact at any instant via Morse forces, so the use of a look-up table speeds the force calculations significantly.*

Another device which improves the speed of calculations is the use of neighbour lists. A particle’s neighbour list is a list of those other particles in the system with which it may interact in the period before the next list update (normally carried out every ~10 timesteps). The neighbour lists are built up by taking into account the potential cut-off distance and the velocity of the fastest particle in the system (which determines the width of the sheath that has to be examined for potential collision partners). The ‘Number of Partners’ parameter which is specified in the Run file defines the maximum allowed size of the neighbour list (i.e. the memory allocation for the list). This parameter is typically set at 50-60, although larger values might be needed on occasions.

Snook does not employ the ‘moving atom approximation’ described in refs. [,], which ignores the effect of small forces (in effect introducing a displacement threshold). The only special treatment that Snook accords to small forces is in connection with the HGE-A algorithm, eqns. 4.5: small pairwise forces can be treated as if they were constant

during the integration timestep, the corrector force being approximated to its value at the start of the timestep. The definition of small forces is selected by the user of Snook (in the Options|Simulation dialog).

4.5. Inelastic processes

Inelastic processes are currently ignored by Snook. This is the reason why the program is unsuited for simulations above 10 keV (when inelastic losses become increasingly important). The author hopes to include some features of this kind in the next release of the Simulation Kit. One reason why this has not been done before now is the difficulty of finding an acceptable way to model the inelastic effects (which are not at all well understood theoretically).

The influence of inelastic events on simulated processes (e.g. sputtering) has been examined in detail recently by Shapiro and Tombrello., They discuss several methods of incorporating inelastic effects into simulation models, and find that in some cases the inclusion of the inelastic effects lead to substantial differences (~30%) in the predicted sputtering yields compared with the elastic model. This is an unexpected result, since for 5 keV Ar bombardment of Cu(100), the calculated inelastic energy losses were only found to be 50-100 eV, or 1-2% of the projectile energy. These inelastic losses are of the same order of magnitude as the energy losses associated with integration errors in the simulation (~0.5%). The influence of inelastic effects on processes such as sputtering remains to be established experimentally.

4.6. Binary targets

Snook is principally designed for use with elemental targets. However, it can be used with binary (or arbitrarily inhomogenous) targets provided that each of the various long-range target-target interactions (X-X, X-Y and Y-Y respectively) can reasonably be described by an identical Morse function. This approximation may well hold good for some common binary alloys or compounds, XY, such as CuNi or GaAs, in which the constituent atoms have similar electronic shells. Snook runs noticeably more slowly with binary targets (which it detects by checking the atomic numbers in the Target file during initialisation), because these involve significantly more computations.

It should be noted that in binary targets, the calculation of the several screened Coulombic interactions (both target-target and target-projectile) will always be handled correctly, regardless of the target composition. Spline functions joining the screened Coulombic potential to the (common) Morse potential will also be appropriate for each target-target interaction permutation (X-X, X-Y and Y-Y respectively).*

The imposition of a single Morse function on all long-range target-target interactions may disqualify the simulation model from use in some applications: for example, preferential sputtering effects in alloys presumably cannot be reproduced by such a model. However, this restriction would probably be irrelevant for the simulation of impact collision ion scattering spectra, which are mainly influenced by the short-range potential.

The simulation of processes in binary targets is another area in which the author hopes to improve the Simulation Kit in the near future. However, for the foreseeable future, there will be a difficulty in obtaining accurate potential parameters for binary solids.

5. Sputtering Simulations

5.1. Introduction

Sputtering simulations at a fixed incident angle are one of the easiest simulations to set up and perform, and they do not make unreasonable demands on computing resources.  Some difficulties arise in the interpretation of results (see below), but general agreement with experimental sputtering coefficients is not difficult to achieve. More work is required for simulations of sputtering involving various projectile incident angles (see chapter 6), but no new principles are involved.

When setting up a sputtering simulation, you need to watch for problems of lattice containment and lattice stability.

5.1.1. Lattice containment

Lattice containment refers to the tendency for the effects of the collision to propagate beyond the boundaries of the finite cluster used in the simulation. If you are using Snook 2.1, this problem can be identified by visual inspection. If you are using Snook 1.2, the check for size effects is to run the simulation on differently sized lattices, and compare the resulting estimates for sputtering coefficients. For sputtering simulations involving keV projectiles you may have to tolerate some lattice containment violations, since to remove them completely may require an unrealistically large lattice.

5.1.2. Lattice stability

Lattice stability becomes an issue in sputtering simulations because of their long time span (300-500 fs normally). Over this time scale, you may see a tendency for the surface layer of the lattice to “relax” towards a configuration which is more stable from the viewpoint of classical mechanics. Relaxation effects are inherent in the assumption of a pair-potential description of the lattice.

Things that can affect this relaxation behaviour are (a) the parameters defining your Morse potential (Model file), (b) the cut-off distance used with your Morse potential, (c) the options controlling the amplitude of thermal displacements and the target temperature, (d) surface structure. A good test for lattice stability is to use a projectile of negligible kinetic energy (0.1 eV, for example) and start it far away from the surface (the starting point is specified as the z0 parameter in the Impact file): this will in effect isolate the lattice. This type of simulation must be carried out with a fixed timestep (which can be selected in Snook’s simulation options.

5.2. Results

5.2. Sputter coefficients

Sputter coefficients (g = number of atoms sputtered per incident projectile) are typically of magnitude 0.1-10 for projectiles in the keV range. A simulation involving 300-500 runs (incident trajectories) can be completed in a few hours on a 100 MHz computer.

Some results are shown below for a 726 atom Ag(111) target bombarded with the following 0.4 keV projectiles: He, Ne and Ar (you can find the input files for the Ne/Ag(111) system in the \examples\ne-ag111 directory of your SK installation). After completion of the simulation, the following filter condition was applied to the output file:

[rw > 0 ] & [vz > 0.0] & [rz > 4.5e-10] & [ke/ep > 0.2]              (5.1)

This condition only selects ejected target atoms with energies above 0.2 eV which are more than 4.5 Å above the original lattice surface boundary. The easiest way to estimate the sputter coefficient is to relate it to the size of the file (FileSize, in bytes) created by the filter operation:

g = FileSize/(RecSize*NRuns),                                                                                         (5.2)

where RecSize is the size of one binary record in .SNK files (40 bytes) and NRuns is the number of runs in your simulation (= 376 in this case). (The screen output of Winnow 2.1 displays this information automatically.)

Similar conditions as (5.1) were applied for a variety of above-surface distances (i.e. zmin = 4.5, 5.5, 7.0, 10.0 and 15 Å respectively, where zmin is the point above the surface where the sputtered atom count commences). The corresponding sputter coefficient estimates are summarised in Table 5.1.

Table 5.1. Calculated sputter coefficients for 0.4 keV projectiles normally incident on 736-atom Ag(111) target (based on 376 incident trajectories). The simulations were terminated at 400 fs, or when all particle energies fell below 1 eV. Experimental values refer to polycrystalline Ag substrates, and are taken from the compilation by Benninghoven et al. (Fig. 2.110).

| Projectile | rz boundary, Å | Sputter coeff., g |
| --- | --- | --- |
| He | 4.5 | 0.84 |
| He | 5.5 | 0.27 |
| He | 7.0 | 0.17 |
| He | 10.0 | 0.10 |
| He | 15.0 | 0.05 |
| He | Experimental | 0.19 |
| Ne | 4.5 | 3.85 |
| Ne | 5.5 | 2.91 |
| Ne | 7.0 | 2.14 |
| Ne | 10.0 | 0.62 |
| Ne | 15.0 | 0.54 |
| Ne | Experimental | 1.6 |
| Ar | 4.5 | 5.66 |
| Ar | 5.5 | 4.37 |
| Ar | 7.0 | 3.19 |
| Ar | 10.0 | 1.93 |
| Ar | 15.0 | 0.95 |
| Ar | Experimental | 2.7 |

The data collected in table 5.1 illustrate how difficult it is to develop a practical criterion which separates the sputtered atoms from others in the plasma-like region which used to be known as the “selvedge” in the SIMS literature of the 1980s.* In these calculations, a range of 4.5 Å was used for the potential cut-off, so this is presumably a lower bound for the width of the selvedge. From comparison with the experimental values, and from visual inspection of the z-distribution of ejected particles, one gets the impression  that the selvedge extends to around 5.5-7.0 Å. It is satisfactory to note, however, that relative yields calculated for the different projectiles are much less dependent on the choice of selvedge width (table 5.2). It is not clear from the literature how the various authors defined their sputter yields in terms of particle positions.

Sputter yields are strongly influenced if a planar surface potential (sect. 4.3.4) is employed in the simulation.

Table 5.2. Calculated ratios of sputter coefficients for 0.4 keV projectiles(He, Ne, Ar)  incident on Ag(111): dependence on assumed selvedge width, zmin (see text for discusssion, and table 5.1 for simulation conditions and ref. to experimental data).

| zmin (Å) | 4.5 | 5.5 | 7.0 | 10.0 | 15.0 | Expl. |
| --- | --- | --- | --- | --- | --- | --- |
| g(Ar)/g(He) | 6.8 | 8.1 | 18.4 | 19.3 | 20.0 | 16.9 |
| g(Ar)/g(Ne) | 1.47 | 1.5 | 1.5 | 1.6 | 1.8 | 1.7 |

5.3. Angular effects in sputtering

There are several distinct kinds of experiments which explore angular effects in sputtering processes. The simulation procedures are generally similar for each kind of experiment, although those simulations where the projectile incident geometry varies are best handled as batch jobs (see the ICISS example in the following section).

A simulation is linked to a particular experimental measurement only by the criteria used at the output-processing stage to categorise sputtered particles. For example, the output data from a given simulation could equally well be processed by Winnow to give a plot of either the azimuthal or the altitudinal distribution of ejected particles (or of many other kinds of distribution): the ultimate use of the output data need not be known or specified at the time that the simulation is run. This flexibility is an important advantage of the Simulation Kit, although it is achieved at the expense of increased complexity for the user.

Winnow’s predefined angular variables (phid, altd, if expressed in degrees, or phi, alt, if expressed in radians) ease the labour of extracting angular information. Suppose you want to make a plot of the azimuthal distribution of ejected particles (excluding the projectile) at an altitudinal angle of 45±3 °. Then you would follow this procedure:

- Filter the output file (a) to exclude records for the projectile (i.e. [rw > 0]), (b) to select only ejected target atoms (e.g. [rz > 4.0E-10] & [vz > 0.0] ), and (c) lastly include only those particles moving at the desired altitude ([altd > 42.0] & [altd < 48.0]).

- Use the Process|Spectrum option to generate the distribution of the variable phid over the angular range 0.0 to 360.0.

The filtering process suggested in (1) can be achieved in a single step:

[rw > 0] & [rz > 4.0E-10] & [vz > 0.0] &

[altd > 42.0] & [altd < 48.0])           (5.3).

The possible range of the angular variable altd is -90.0 to +90.0 (degrees), but the negative values (implying motion into the bulk) are rarely needed.

Positive values of altd imply positive values of vz (the perpendicular velocity), which means that the term [vz > 0.0] in equation 5.3 can be omitted, since it is redundant.

6. Impact Collision Ion Scattering

6.1. Introduction

The repulsive interaction between an incident atomic projectile and a target atom at the surface of a lattice creates a region (known as the shadow cone) into which the primary projectiles cannot penetrate. Impact Collision Ion Scattering Spectrometry (ICISS) is an ion scattering technique which uses the shadow cone effect to derive structural information.Error: Reference source not found The shadow cone formalism has also been applied to explain projectile-incidence anisotropies in other techniques such as SIMS,, ion-induced secondary electron emission, and ion-induced Auger electron emission.,

incident projectiles

shadow cone region

Figure 6.1. Shadow cone region formed by trajectories of projectiles scattered by repulsive potential.

The simulation of measurements made in ICISS experiments is one of the most demanding applications of the Simulation Kit. This is chiefly because ICISS measurements only detect backscattering of projectiles into a narrow angular range (~ 1°). For this reason, practical ICISS simulations require the use of simplified scattering models and a modest number of target atoms. Using these simplified models, a wide-range ICISS spectrum can be simulated in approximately one working day on a Pentium PC.

The simulation of ICISS data and other kinds of angular scans are good candidates for running as batch processes (using Snook’s batch run option), as described in section 6.2.

In ICISS experiments, a projectile is incident parallel to a row of the target lattice. Scattered projectiles are detected in the same plane, at a scattering angle of ~180°.Error: Reference source not found Scattering in the plane requires that the projectile’s collision partners all be coplanar, so the first simplifying assumption we can make is to consider only those collisions involving projectiles incident in the plane of the atomic row. This model implicitly ignores the effects of vibrational displacements. A related simplification is to consider a target which consists only of atoms in the plane of incidence (see fig. 6.2). Finally, since ICISS is highly surface sensitive, we need only use a target consisting of a small number of atomic layers (3 or 4).

Using the first of these assumptions (and, optionally, the second and third) we can then determine the relative backscattering yield by considering only those projectile trajectories which lie in the plane of the atomic row (see fig. 6.2).

The following discussion assumes that the parameters of the projectile-target interaction potential (notably the screening length correction) are known. In practice, these will often have to be determined heuristically with reference to a sub-set of the experimental data. Fitting exercises of this kind should usually be carried out using a “binary search” method: for example, suppose the screening length correction is known to lie in the range 0.7 to 0.9, and the optimum value is 0.84, then the search would proceed by testing the following values: 0.8 (intermediate between 0.7-0.9), 0.75 (fails), 0.85, 0.825 ...

6.2. ICISS example project

The input files you will need for the example ICISS project can be found in the \examples\iciss directory of your SK installation. Be sure to read the readme.txt files in that directory and in the \examples directory respectively before you attempt to run the simulation (which uses 2 batch files).

The input files for each scattering geometry are identical, with the exception of the Run files. The latter differ only in the specification of the projectile altitudinal angle of incidence, so it is easy to generate Run files for each geometry from a common ‘template’ Run file simply by editing the altitudinal angle field (using Spider’s Run|Open command).  If you subsequently need to modify some other field in the Run file (e.g. the timestep) you may find that it is less tedious and error-prone to go back and generate the files you need from a common ancestor template, rather than edit the existing Run files.

Fig. 6.2 illustrates the scattering geometry on a 2-dimensional target lattice. In this example we shall consider the ideal case of 180° scattering.

1 keV He

4.4271 Å

<112>

2.556 Å

(100)

Figure 6.2. 26-atom Cu(110) lattice used for example project (projection in xz plane). The arrows depict the range of incident projectile trajectories in the simulation.

Fig. 6.3 shows the processed results of the ICISS simulation, namely a plot of the 180° backscattering yield as a function of the projectile incident altitudinal angle. The backscattered projectile yield is extremely low (< 100 counts in the 2° counting interval per 8000 trajectories) over most of the angular range. These data are produced by the following method.

First, in the Run file of the ICISS project, it was specified that only emitted projectile data should be written to the output file. (This restriction is not mandatory, as filtering can separate the projectile data later, but it saves unnecessary disk usage.) Second, after the simulation was completed, the resulting output file for each incident angle was filtered to select out the angular fraction of interest e.g. for 15° incidence:

[altd > 14.0] & [altd < 16.0] & [vx > 0.0]                                                           (6.1)

Figure 6.3. Calculated direct impact backscattering yield for 1 keV He projectiles incident on Cu(110) along a <112> azimuthal direction (see fig. 6.1). Yields are calculated at either 3° or 1° intervals, using a 2° acceptance range for the altitudinal angle (e.g., the point at j = 60° refers to the fraction of particles backscattering between 59-61 °). The lines are drawn to guide the eye, and may be misleading at the sharp edges. Each point is based on 8000 runs (trajectories). See fig. 6.4 for an analogous simulation which incorporates thermal vibration effects.

This filter selects particles scattered in the +x direction at altitudinal angles (expressed in degrees) between 14-16° i.e. at an angle of 15±1°. (The symbol altd refers to the altitudinal angle expressed in degrees.) Since the output routine only stored projectile data in this case, there is no need to select projectile particles explicitly (i.e. by using the additional filter condition: [rw = 0]).

Fig. 6.3 illustrates the existence of ‘critical angles’ jc for 180° (‘direct impact’ or centre-to-centre) collisions, which are indicated by sharp onsets in scattering intensities. These appear as peaks because there is a concentration of trajectories at the shadow cone edges.

There are several further points to note about  fig. 6.3.

- Initially, I ran a survey ‘scan’ across the entire angular range at 3° intervals. Then I ran a few more simulations at 1° intervals around the sharp edges. In a real research project, you would complete the process by getting more data at 0.5° intervals or less in the vicinity of the edges (which are very sharp).

- A real project would reduce the width of the angular counting interval to correspond to that used in the experiment, and would include vibrational effects (see next item).

- Since no vibrational effects are incorporated into this model, the critical angles jc corresponds to the edge onsets, rather than their “85% heights” as in the experimental case (see section 6.4).

- If the experimental data refer to scattering at some angle < 180°, then the filter condition should be modified accordingly. For example, for 140° degree scattering, condition 6.1 (for j  =15° ) must be modified to: [altd > 54.0] & [altd < 56.0] & [vx > 0.0] (N.B. use a filter [vx < 0.0] if the scattered projectiles move in the negative x direction, as they would if j > 50° in this example).

The critical angles, jc, can be estimated from fig. 6.3 as ³ 15° and ³ 52° respectively; further simulations (not included in the example project) were used to refine the lower of these figures, and gave jc = 15.32 ± 0.02°.

The cone utility (found in your sk\bin directory) can be used to check the general correctness of the critical angle estimates. The cone calculations use a 2-body approximation to the scattering system. The projectile-target interaction is of ZBL type, with a screening length correction of 1.0 (i.e. uncorrected). The cone estimates turn out to be (a) 14.7° for the in-row scattering event (d=4.4271 Å), and (b) 51.5° for the surface-nearest neighbour scattering event (2.556 Å) respectively (after adding in the 30° inclination of this row to the surface).

6.3. Angular conventions

There is some possibility for confusion in the angular conventions used by the Simulation Kit. The angular variables used by Winnow for filtering and other options are based on a self-consistent system (see Winnow on-line Help for details). The user-programmed option in the Run file dialog of Spider also follows the same system. However, the projectile incident altitudinal and azimuthal angles are defined in the Run file in a manner similar to what is used by experimentalists: these angles actually define the orientation of the position vectors that join the surface ‘impact point’ to the starting projectile position. For this reason, the incident altitudinal angle is always entered in the Run file dialog as a positive number, although strict geometry would indicate a negative value (since vz/Ö(vx² + vy²) < 0).#  For 180° scattering, the projectile incident and scattering angles would both therefore be treated as positive quantities (by Snook/Spider and Winnow respectively).

6.4. Vibrational effects in ICISS

The ICISS example just described does not incorporate the effects of thermal vibrational displacements in the target lattice. These displacements increase the computational problem immensely, because of the now small probability of occurrence of a direct-impact event (which requires a scattering configuration in which both scattering atoms to lie coaxial to the projectile path).

A quantitative investigation of this problem, using again the 1 keV He-Cu(110) geometry considered in 6.2, indicated that the backscattered yield at the 16° maximum (fig. 6.3) would be about 50 events per 100,000 trajectories for a counting range of ± 2° altitudinal and ±10° azimuthal. To express this in computational terms, the inclusion of 3-dimensional thermal displacement effects lengthens the computation time required for comparable counting statistics by 3 orders of magnitude compared with the no-vibration model, thereby making them impractical for routine purposes.

In order to work around this problem, Snook offers the option of suppressing thermal vibrational displacements in the y-direction, while applying them in the normal way along the x- and z-directions. (This approach relies on the assumption that the projectile’s azimuthal incidence is parallel to the x-axis, which is achieved by leaving this parameter at its default value of zero in the Run file.) Thermal displacements applied in this manner maintain the coplanarity of lattice atoms in the projectile plane of incidence. To activate this option in Snook, it is only necessary to select the “No y-vibration” item which appears in the Simulation Options dialog box (on the Options menu).†

Figure 6.4. Calculated direct impact backscattering yield for 1 keV He projectiles incident on Cu(110) along a <112> azimuthal direction. These yields were determined (from 8000-trajectory simulations)  in the same manner as those in fig. 6.2, with the difference that x- and z-thermal vibrational displacements were incorporated into the model used here.

Fig. 6.4 presents a reprise of the ICISS simulation described in figs. 6.2 and 6.3, with the difference that now x- and z- axis vibrational displacements are incorporated into the simulation model. The resulting data no longer feature sharp peaks at the critical angles. Instead, these are broadened by convolution with the vibrational displacement function. The FWHM of 5.5° is similar to what is observed experimentally (for 2 keV He -> Cu(110)).

The total area under the peaks in fig. 6.4, which represents the total scattering yield, remains the same as in fig. 6.3. The statistical uncertainty of the simulated yields can be estimated via Poisson statistics as ÖN = ca. 10% for the peaks shown in fig. 6.4.

Fig. 6.3 demonstrated that the critical angle for the first peak in this scattering geometry lies above 15°. Further investigation placed it in the range 15.32 ± 0.02°.‡  This angle corresponds to the peak maximum in fig. 6.4. The usual experimental practice of reading the critical angle position at 85% of the intensity maximum would yield an estimate of about 14.5° if applied to fig. 6.4 (not a particularly good agreement).Error: Reference source not found The level of agreement between these two estimates might improve if better statistics were obtained in the simulation (e.g. by using 20,000 trajectories rather than 8000), or if the separating intervals between angular samples were reduced.

7. Depth Distributions

7.1. Introduction

This chapter discusses a project which looks at the vertical displacement and spatial distribution of target particles during ion bombardment.

The input files used for this project can be found in the \examples\mixing directory. The project requires about 4 hours to run, and generates a very large output file (dynvars.snk) which is 24 MB in size. The project demonstrates the ability of the Simulation Kit to use a generic output file for extracting different kinds of information about a system.

Briefly, the physical system consists of a Cu(100) target which is bombarded at normal incidence by 0.5 keV Ar projectiles. The target consists of 12 atomic layers, each of which contains 225 Cu atoms (15x15x12 = 2700 atoms in total). This relatively large target was chosen in order to ensure that the collision was contained both laterally and vertically. The simulation uses 225 runs. Each trajectory is followed until either the fastest atom in the system has kinetic energy below 2 eV, or 500 fs has elapsed (in practice, the first condition is met within 300-400 fs). After the termination of each run, state information (r, p, ...etc) is recorded for each particle in the system.

The purpose of the simulation was to examine (a) the extent of vertical mixing of atoms from different layers; (b) the depth of origin of sputtered atoms; (c) the vertical distribution of implanted primary projectiles; (d) the formation of Wehner spots.

For purposes (a) and (b), the first necessary stage in data processing was to filter the output file, dynvars.snk, in a way that would isolate the contributions from individual layers. This can be achieved (using Winnow’s Filter option) for layer 1 by noting that this layer corresponds to the particles referenced by lines 1-225 of the input Target file mixing.trg. Thus a suitable filter expression for this layer is as follows:

[rw > 0] & [rw <= 225]                                                                                          (7.1)

Likewise, the following filter can be used to isolate the second layer:

[rw > 225] & [rw <= 450]                                                                                     (7.2)

After several applications of this procedure, one obtains a series of smaller files layer1.snk, layer2.snk, ... which contain the layer-specific data.

7.2. Sub-surface depth distribution

Fig. 7.1 shows the post-bombardment depth (z) distribution of particles plotted according to the layer of origin. The ordinate scale consists of the total counts measured for each channel (1 channel = 0.4 Å) summed over all 225 runs. The individual curves were obtained by using the Processs|Spectrum option in Winnow, with a “spectrum independent variable” defined as rz*1.0E10 (i.e. the z-position expressed in Å).

The integrated area under each curve is identical, and corresponds to (225 runs x 225 particles = ) 50625 counts. The most obvious feature of fig. 7.1 is the relative broadening of the curves in the outermost layers (with consequent decline in heights of the curve maxima). It is possible to discern tails on the low-z side of these curves which are indicative of recoil mixing. The extent of mixing is somewhat disguised by the contribution from particles near the edges of the target planes, which are little disturbed by the collision cascade. We shall not discuss further how to present data of this kind in order to better bring out the mixing effects, although clearly it is a delicate problem.

Figure 7.1. Depth distribution of target atoms in Cu(100) following 0.5 keV Ar bombardment. The figure shows the contribution (total counts from 225 runs) from particles in each of the first 7 atomic layers (225 particles per layer; 0.4 Å per channel).

7.3. Origin of sputtered atoms

Fig. 7.2 isolates the depth distribution of particles originating in layer 4 (5.4 Å) of the target. These data are the same as those shown in fig. 7.1, but now the ordinate scale has been expanded by a factor of 100. From fig. 7.2 it is evident that a negligible number of atoms from layer 4 are able to penetrate the surface (at z = 0.0 Å) and leave the target as sputtered atoms.

Figure 7.2. Same data (for layer 4 only) as shown in fig. 7.1, but with the ordinate scale expanded by a factor of 100. Note the total absence of layer 4 atoms found at z > -1 Å.

The depth of origin of sputtered atoms can be quantified by the following methodology. First some threshold z value must be defined which represents the beginning of the region occupied by sputtered particles, as opposed to the outermost surface atoms. As discussed in section 5.2, this definition is not a trivial task. For the purposes of discussion, we shall assume a threshold here of z = +6.0 Å. The contribution of each atomic layer to the sputtered atom yield can then be estimated by performing a further filter operation on the processed output files (layer1.snk, layer2.snk etc.) whose preparation was described above. This filter saves those particles which are located at vertical (z) positions at 6 Å or greater:

[rz > 6.0E-10]                                                                                                              (7.3)

The finding of this procedure is that of a total of 1080 atoms found to be sputtered after 225 runs (a mean yield of 4.8 atoms per incident projectile* ), 1067 of these (99%) originated from layer 1, 13 (1%) originated from layer 2, and none originated from the deeper layers (3, 4, 5...). This result emphasises the surface specificity of sputtering.

7.4. Projectile implantation profile

The projectile depth (z) distribution can be accessed from the output file (dynvars.snk) of the project by filtering with the expression [rw = 0], followed by generation of the depth “spectrum” as described in section 7.2. Since the projectile may have left the target region (which covers a radius of ca. 17 Å from the origin), it is prudent in practice to use a filter condition like the following, which ensures that the filtered records correspond to projectiles located in the vicinity of the target:

[rw = 0] & [sqrt(rx*rx + ry*ry) < 17.0E-10]                                         (7.4)

Figure 7.3. Depth (z) distribution of projectile species at termination of the simulation outlined in section 7.1 for 0.5 keV Ar-Cu(100). The figure does not include those projectiles (5.8% of total) that were laterally displaced by more than 17 Å from the lattice origin. The width of each channel is 0.8 Å.

Fig. 7.3 shows the plot which results from this procedure. The distribution is noisy because it is based on only 225 trajectories. A substantial fraction of the projectiles are reflected from the surface (ca. 20%, depending on where the surface is drawn). Of more interest to the present discussion are those projectiles which remain lodged inside the target. Fig. 7.3 shows that these are mostly stopped within 8 Å of the surface (equivalent to 4-5 Cu(100) atomic layers).

7.5. Wehner spots

It has long been known that sputtered particles are preferentially ejected along directions parallel to the more closely-spaced atomic rows.  This can be observed experimentally in the phenomenon of “Wehner spots.”Error: Reference source not found Although this topic is unconnected with the subject of the chapter, it can be examined using a similar methodology, and is for that reason briefly discussed here. Wehner spots are prominent features in the images formed by sputtered particles incident upon photographic plates (or other imaging devices) facing the plane of the surface under bombardment (a set-up similar to that used for Laue or electron diffraction).

Wehner spots can be investigated through simulations by making plots of the asymptotic sputtered particle momenta in the (x,y) plane. Fig. 7.4 shows a plot of px versus py for sputtered particles in the 0.5 keV Ar-Cu(100) system. These were obtained by first filtering the output file (dynvars.snk) with the following expression, which isolated the sputtered target atom component:

[rw <> 0] & [rz > 6.0E-10]                                   (7.5)

The SNK file that resulted was then loaded into the trajectory plotter, from which fig. 7.4 was copied.* Fig. 7.4 constitutes a projection of the particle trajectories onto the (x,y) plane. The maximum spot intensities coincide with 45° emission along the <001> azimuthal directions, which correspond to the altitudinal directions parallel to <011> atomic rows.

Figure 7.4. Scatter plot of sputtered particle momenta (px vs. py) for 0.5 keV Ar-Cu(100) system, showing the formation of Wehner spots. The x and y axes coincide with <011> target lattice rows. The diagonal directions lie parallel to <001> rows.  The figure is based on 225 projectile trajectories.

This example will not be discussed in greater detail. An obvious first improvement of the data presented in fig. 7.4 would be to exploit symmetry and “fold over” each data point into symmetrically equivalent quadrants. This can be achieved (with some difficulty) using the Format Columns option of Winnow,‡ but it would better to program the operation directly in a spreadsheet or computer program.

8. Thermal Vibrations

8.1. Introduction

The author became increasingly dissatisfied by the method of handling thermal vibrations in the first version of the Simulation Kit. The original intention was to provide an order-of-magnitude estimate of lattice atomic displacements. As the following discussion will show, the choice of atomic displacements for atomic collision simulations is a non-trivial problem. However, feedback from users indicated that a more ‘precise’ treatment was necessary, particularly for  temperature variation studies. Accordingly, the basis of the computations made by Snook and Spider (in versions 1.2 or 2.1) is outlined in some detail here. The discussion in this chapter, which is certainly not the only way to approach the problem, should allow users to decide for themselves on the value of the treatments used.

Before discussing the equations, the following general points are worth drawing attention to:

- Users can always override the mean square thermal vibrations calculated by Spider by simply editing the values in the .MDL file. (Refer to the File Formats topic in Spider’s online Help)

- Most experiments based on keV ion bombardment are not particularly sensitive to thermal vibration effects.

- Errors in the treatments of vibrational effects will only significantly influence simulations of physical processes if the root mean square (rms) vibrational amplitude error is significant in comparison with the scattering cross section (collision radius).

- Most published keV bombardment simulations have not included thermal vibration effects.

- Don’t confuse the rms vibrational amplitude Ö<r2>, with the mean square thermal vibrational amplitude, <r2>.

- Don’t confuse the isotropic mean square amplitude <r2> with the unidirectional mean square amplitudes <x2>, <y2>, <z2>.

- Debeye temperatures are uncertain to at least ±10%, and often more.

8.2. Theory

According to the Debeye theory, the isotropic mean square thermal vibration amplitude, <r2>, of atoms in a monatomic solid is given by the following expression:

<r2> = 9h 2 T/(MkqD2). [ f(qD/T) + qD/4T]                                                                     (18.1)

where qD is the Debeye temperature, k is Boltzmann’s constant, T is the absolute temperature and M is the mass of atoms in the solid. f(x) is the following function, where x = qD/T :

x

f(x) = (1/x).ò ydy/(ey - 1)                                                       (18.2)

0

Equation 18.1 refers to the mean square amplitude of an isotropic oscillator. This is the quantity which Snook reads from the .MDL file created by Spider. For vibrations in a specific direction (x, y or z), the right hand side of equation 18.1 must be divided by 3:

<x2>  = <y2> = <z2> = <r2>/3 = 3h 2 T/(Mk qD2) . [ f(qD/T) + qD/4T]                        (18.3)

These unidirectional amplitudes are the quantities which are calculated and used by Snook.

If we substitute for the physical constants, and interpret M as the mass in atomic mass units (amu), then with the substitution (x = qD/T) the expression (18.3) simplifies to:

<x2> = 145.5/MqD . [ f(x)/x + 1/4]   Å2                                                                         (18.4)

[h = 1.05459E-34 Js; k = 1.38066E-23 JK-1; 1 amu = (1000NA)-1 = 1.6606E-27 kg]

8.3. Comments on the Literature

There is some confusion in the equations published in the literature, in that equation 18.3 is sometimes associated with the isotropic vibrations (more properly described by equation 18.1). This presumably occurs because of typographical errors. Despite this, and other discrepancies, most authors seem to broadly agree on the constant pre-factor (5.5) in equation 18.4 (although physicists tend to calculate the pre-factor using nuclear rather than atomic masses, giving a value of 6.0). The reader is urged to be cautious when reading the literature on this subject (including this article!).

8.4. Implementation  in Spider and Snook

Spider (versions 1.2 and 2.1) uses equation 18.4 to calculate <x2> for bulk and surface atoms, based on the values for T and qD specified by the user (for the Model file), and the approximations to f(x) discussed below. These unidirectional <x2> values are read by Snook from the Model file. The displacements added to each lattice atom x, y and z coordinate are calculated (using the Box-Muller method) to be consistent with a random distribution having Gaussian deviates characterised by a standard deviation <x2>. Bulk, surface parallel and surface perpendicular displacements respectively are drawn from distributions with different standard deviations. For purposes of the vibrational correction, a lattice atom is classified as a ‘surface atom’ if its z-coordinate, z[n], places it above the anchor atom at z[1], or no more than 1 Å below it:

if z[n] >= z[1] - 1.0E-10 then {the atom is in a surface site }

else { atom is in a bulk site }

The method for calculation of the Debeye function f(x) in equation 18.4 is now explained.

f(x) can be calculated by numeric integration of equation 18.2. Eckstein provides the following approximation:Error: Reference source not found

x

f(x) = (1/x).ò ydy/(ey - 1) ) = (1 - x/4 + x2/36 - x4/3600 + ...)                      (18.5)

0

This approximation is used by Snook for x in the range 0 to 3.0, i.e. in the range T = 0.33qD to “infinity”. Recall that x = qD/T.  For “infinite” T, f(x) = 1.0; for T = 0.33qD, f(x) = 0.48.

As T ® 0, so too  f(x) ® 0. For values of x  > 3.0 the following (low-temperature) approximation is used:Error: Reference source not found

f(x) = [2/6  - (x + 1)exp(-x) - (x/2 +1/4)exp(-2x) - ...]/x                            (18.6)

For x >> 1, f(x)/x becomes small in comparison to the ¼ term in equation 18.4, and the lattice vibrations approach their zero-point levels.

It is worth pointing out that these thermal displacements are computed without reference to the potential function (Morse) used for the target-target interactions in your lattice. This means that the (thermo-)dynamical temperatures associated with your lattice (which depend on the potential parameters) will not correspond to the temperature which you specified in the model file. The true lattice temperature is related to the mean difference in potential energies, <V1 - V0>, of the lattice with (V1), and without (V0), thermal displacement effects applied. For a system of oscillators interacting via quadratic (harmonic) terms, theory suggests a value of <V1-V0> = 3/2kT (which we shall use as an approximation for the Morse potential too; this result comes from either the Equipartition principle, or the Virial theorem,Error: Reference source not found depending on your point of view). The discrepancy between theory and the values returned by Snook depend on the potential function used by Snook. For example, the default Morse potential for Cu gives a value of <V1 - V0> = 0.076 eV (300 K), compared with 3/2kT = 0.04 eV.

Further discussion of these matters would take us too far afield, but the conclusion must be that the choice of an appropriate thermal displacement depends to some extent on the purpose of the simulation. For a study of ion scattering or channelling, the primary focus of interest is the displacement itself. However, for study of lattice dynamics (melting, film growth, diffusion etc.), the potential energy associated with (or implied by) the displacements should be the primary consideration. Sputtering - one of the main applications of such simulations - unfortunately falls in the area between these extreme cases.

8.5 Lattice atom velocities

The simulation options dialog box in Snook allows the user to initialise the velocities of the target lattice atoms. The idea is to set-up the target lattice as a dynamical system of a specific temperature (see, however, the preceding section for qualifications to this remark). The velocities are randomly applied, according to a Maxwellian distribution.‡ Note that this velocity initialisation is based on a classical theory, in contrast to the quantum mechanical foundation of the Debeye theory on which the thermal displacements are estimated.

According to the Equipartition Principle (from statistical mechanics), the mean translational energy of an atom in the classical limit is 3/2 kT<KE>  (k is the Bolzmann constant). This can be broken down into contributions of ½kT for each direction of motion. The velocity distribution is deduced from the following assumptions:

(a) atomic energies are distributed according to a Boltzmann distribution: i.e. P(E) = exp-((KE + V)/kT) = exp(-KE/kT).exp(-V/kT);

(b) atomic positions and momenta are distributed independently: i.e. P(E) = P(V).P(KE).

One advantage of applying these velocities is that they appear to increase the stability of the isolated (unbombarded) lattice over long time periods (~ 500 fs).#

References

Wolfgang Eckstein, ‘Computer Simulation of Ion-Solid Interactions,’ Springer-Verlag, Berlin, 1991.

E.S. Mashkova and V.A. Molchanov, ‘Medium-Energy Ion Reflection from Solids,’ North-Holland, Amsterdam, 1985.

D.E. Harrison Jr., ‘Sputtering Models - a Synoptic Review,’ Radiation Effects, vol. 70 (1983) 1-64.

R. Smith and D.E. Harrison Jr., ‘Algorithms for Molecular Dynamics Simulations of keV Particle Bombardment,’ Computers in Physics, vol. 3 (1989) 68-73.

M.T. Robinson, Theoretical Aspects of Monocrystal Sputtering, in Sputtering by Particle Bombardment I (pp. 73-144) , R. Behrisch ed. (Springer-Verlag, Berlin, 1981).

H. Niehus, W. Heiland and E. Taglauer, ‘Low-energy Ion Scattering at Surfaces,’ Surface Science Reports vol. 17 (1993) 213-303.

Herbert Goldstein, Classical Mechanics, 2nd ed. (Addison-Wesley, Reading MA, 1981).

L.D. Landau and E.M. Lifshitz, Course of Theoretical Physics, Volume 1: Mechanics, 3rd ed. (Pergamon, Oxford, 1976).

H.S. Tan and M.A. Karolewski, Nuclear Instr. Meth. B73 (1993) 163.

M. Gryzinski, Phys. Rev. 138, (1965) A305.

M.A.D. Fluendy and K.P. Lawley, Chemical Applications of Molecular Beam Scattering (Chapman and Hall, London, 1973).

W.J. Duffin, Electricity and Magnetism, 4th ed. (McGraw-Hill, New York, 1990).

M. T. Robinson and I.M. Torrens, Phys. Rev. B9 (1974) 5008.

J.F. Ziegler, J.P. Biersack and U. Littmark, The Stopping and Range of Ions in Matter, vol. 1 (Pergamon, New York, 1985).

R.S. Daley, D. Farelly and R.S. Williams, Surface Sci., 234 (1990) 355; S.J. Anz and R.S. Williams,  Surface Sci., 372 (1997) L323.

W.H. Press, B.P. Flannery, S.A. Teukolsky and W.T. Vetterling, Numerical Recipes in Pascal (editions also in Fortran or C) (Cambridge University Press, Cambridge, 1989).

Derek Wood, Data Structures, Algorithms and Performance (Addison-Wesley, New York, 1993).

Robert Sedgewick, Algorithms (2nd Ed.), (Addison-Wesley, Reading, MA, 1988), chapter 26 (“Range Searching”).

J.W. Rabalais, H. Bu and C.D. Roux, Phys. Rev. Lett. 69 (1992) 1391.

K. Broomfield, R.A. Stansfield and D.C. Clary, Surface Sci. 202 (1988) 320.

R. Feynman, Lectures on Physics, Vol. II (sect. 39-5), Addison-Wesley, Reading, MA, 1964.

N.W. Ashcroft and N.D. Mermin, Solid State Physics, (Saunders College, Philadelphia, 1976).

R.A. Gibbs, S.P. Holland, K.E. Foley, B.J. Garrison and N. Winograd, Phys. Rev. B24 (1981) 6178.

D.E. Harrison, Crit. Rev. Solid State Mater. Sci. 14 S1  (1988) 1.

R. Smith, D.E. Harrison and B.J. Garrison, Phys. Rev. B40 (1989) 73.

M.H. Shapiro and T.A. Tombrello, Nuclear Instr. Meth. B 94 (1994) 186.

M.H. Shapiro and T.A. Tombrello, Nuclear Instr. Meth. B 102 (1995) 277.

A. Benninghoven, F.G. Ruedenauer and H.W. Werner (eds.), Secondary Ion Mass Spectrometry (Wiley, Chichester, 1987).

M.J. Witcomb, Radiat. Eff. 27 (1976) 223.

C.C. Chang, Surf. Interface Anal. 15 (1990) 79.

B.A. Brusilovosky, Vacuum 35 (1985) 595.

L. Wong, P.F.A. Alkemade, W.N. Lennard and I.V. Mitchell, Nucl. Instr. Meth. B45 (1990) 637.

R. Pfandzelter and J.W. Lee, Nucl. Instr. Meth. B45 (1990) 641.

R. Spitzl, H. Niehus and G. Comsa, Surf. Sci. Lett. 250 (1991) L355.

C. Kittel, Elementary Statistical Physics, (John Wiley, New York, 1958).

* These relationships are well-known to players of Snooker (from which Snook takes its name) and croquet.

* The derivatives  are evaluated by means of  ‘chain-rules’ like the following: ¶U(r12)/¶x1 = ¶r12/¶x1*¶U(r12)/¶r12 = (x1-x2)/r12*¶U(r12)/¶r12, where r12 = ((x1-x2)2+(y1-y2)2 +(z1-z2)2)½. Terms like (x1-x2)/r12 are known as ‘direction cosines’.

* http://www.simtel.net/pub/simtelnet/

* One can draw a rough analogy here with the ambiguity which sometimes arises in interpreting LEED patterns purely on the basis of the kinematic theory.

* If you rotate the lattice in the yz or zx planes you will normally have to edit (cut/paste) the resulting Target file to ensure that your desired anchor atom coordinates are found in the first line of the new file.

‡ The projectile altitudinal angle of incidence is a “polar” angle which takes values between +90° (normal to surface) and 0° (parallel to surface). The projectile azimuthal angle of incidence is the angle subtended around the z-axis in the xy plane; it is zero for incidence parallel to the x-axis, and 90° for incidence parallel to the y axis. The angular convention used by Winnow (and Spider’s “User-Defined” Run file output option) uses (unlike here) a strict sign convention based on the direction (sign) of vector components. You should consult the Winnow on-line Help for details.

* However, even under these circumstances, redundancies do not generally occur in the Impact files generated by Spider, because of the character of the underlying generating algorithm (see Spider Help for more on this topic). Here is a simple numeric illustration. For 5 impacts along an axis of length a, the impact points generated by Spider will be 0, a/5, 2a/5, 3a/5, 4a/5 while for 5 impacts along an zone axis of length 2a the impact points will be: 0, 2a/5, 4a/5, 6a/5, 8a/5; if the lattice repeat length is a, the latter series is equivalent to: 0, 2a/5, 4a/5, a/5, 3a/5, i.e. there is no redundancy when the impact points are folded back inside the true reduced impact zone.

* These coefficients of the screened Coulombic potentials can be found in the review articles cited in the Introduction, or in the Spider online Help.

* The reasoning is as follows.Error: Reference source not found (a) The energy required to break N bonds and remove a single atom from the bulk into the gas phase should be roughly twice the cohesive energy, since atom formation by bulk vaporisation creates on average two gas phase atoms for every N broken bonds, where N is the coordination number (i.e. EB ~ 2U0); (b) the surface binding energy should be roughly half of the bulk binding energy (i.e. Es = 0.5EB). Hence, ES ~ U0.

* Floating point calculations, especially those involving the exponential or other functions, are extremely time-consuming.

* One of the main reasons why Snook runs slowly with inhomogenous targets is that the spline function coefficients have to be recalculated many times. It is prudent, for this reason, to define spline limits which are as narrow as possible, so that particles spend less time in the spline region.

* It is not my intention to assert the physical existence of the “selvedge.” But the term is a reasonable description of the messy surface region which assembles in classical bombardment simulations. Some writers, e.g. Robinson [Error: Reference source not found], use the term as a synonym for “surface”.

# Spider does not even allow you to enter a negative altitudinal angle.

† The “thermal vibrations” option must, of course, be enabled in the Model file used by the simulation. Otherwise the option discussed here has no effect.

‡ The peak structure at the leading edge was also found to be considerably sharper than implied by fig. 6.3, with the intensity reaching 470 counts at j = 15.35°.

*M. Hou and W. Eckstein, Nucl. Instr. Methods B13 (1986) 507, calculated a sputter yield of 2.6 for the same system. The discrepancy with the value given here is probably related to the definition of the sputter threshold, as previously discussed in section 5.2.

* In the Display|Options dialog, “Momenta” was chosen.  The plot shown in fig. 7.4 was created by playing through all frames, using “Points” for the symbols, and by having the “Refresh between frames” option unchecked.

‡ You need to make 4 data (*.dat) files, using the abs() function to permute the sign of the px, py entries in turn as (+,+), (+,-), (-,+) and (-,-) . The files can be recombined using an editor. There will be considerable redundancy (duplicate entries) but this won’t affect the resulting plot.

‡ No attempt is made to ensure that <vx>, <vy> and <vz> are precisely zero, although in practice this very nearly happens in large lattices, as expected.

# To carry out a simulation on an isolated lattice (no incident projectile), you need to specify a zero projectile energy (Projectile file), and a zero termination energy (Model file); it is also advisable to set a large z0 parameter (in the Impact file) so that the ‘stationary’ projectile does not interact with the lattice. Lastly, you should set a timestep of ~ 1 fs (Model file) and specify that it remain fixed throughout the simulation (in Snook’s Simulation options dialog box). For a simulation of this sort, the parameters in the impact file (other than z0) are irrelevant.