ALIGN

Version 1.6 for the IBM PC

5/23/84

(c) Copyright John L. Dickinson 1983, 1984

COPYING PROVISIONS

This program and documentation may be freely copied and shared

provided it is not modified in any manner and this notice is not

bypassed or removed.  Please keep the ALIGN.DOC documentation file

on the same diskette with ALIGN.EXE.

If you find the ALIGN program useful, a contribution will be

appreciated.  ($20 suggested.)  Comments or suggestions concerning

ALIGN are welcome at any time.

John L. Dickinson

2470 N. Tierra Verde Pl.

Tucson, Arizona 85749

PROGRAM OVERVIEW

The ALIGN program allows you to move a diskette drive head to

absolute track locations and operate the drive while checks and

adjustments are made.  40 track single or double-sided drives are

supported.

ALIGN is one of several tools needed to adjust and service

diskette drives.  The ALIGN program does not attempt to teach you

how to do an alignment, but it isn't difficult if you have what

you need.  In addition to this program, you should have:

The drive manufacturer's alignment and service procedures.

An oscilloscope (dual trace with triggered sweep).

A DYSAN double-sided analog alignment diskette for 40 track

drives, 224/2A #800180.

Extension or external diskette drive signal and power cables are

very helpful.

See the notes at the end of this document for possible sources for

these items.

An analog alignment diskette contains NO data, just special

waveforms and can't be copied.  (It's really copy protected!) You

can try the program using any diskette.  The ALIGN program does

not write on diskettes.

(It should also be possible to use this program to adjust 80 track

drives provided you have the proper alignment diskette.  However,

some of the program commands may not be useful since they have

track addresses hard coded for the 40 track alignment diskette.)

ALIGN Version 1.6                      Page 2

SYSTEM AND DISKETTE DRIVE REQUIREMENTS

ALIGN runs under IBM PC-DOS on a PC or PC/XT with at least 64K and

an 80 column display, either monochrome or color.

Since ALIGN uses standard BIOS interfaces for all I/O, it should

also work on most IBM compatibles.  It does not work properly on

the PCjr.

The diskette drives to be adjusted are connected to the PC in the

usual manner.  The ALIGN program works with standard IBM (Tandon)

diskette drives and should work with others that use the same

electrical interface.

USING THE ALIGN PROGRAM

Enter ALIGN after the DOS prompt and press any key to get past the

initial screen to the main command menu.  Chances are, you'll be

able to use the program without reading any more of this

documentation.

Commands are entered one at a time on the command line.  All

commands may be abbreviated and the first letter is sufficient for

all but one command.  (INFO requires the 3 letters, INF.)

Some commands require one operand as listed on the menu.  Numeric

operands, such as track number or seconds, are decimal.  The first

track is track 0, not track 1.

If the diskette drive under test is running, pressing any key will

cause it to stop in about 2 seconds.  This key may be the first

letter of the next command or any key such as the RETURN key.  The

drive will stop by itself after 60 seconds.

If the drive is stopped and and you want to continue the previous

motion command, simply press the RETURN key.  This enters a

null command which is interpreted just like a GO command.

Commonly used commands may also be entered using the program

function keys as shown on the command menu.

ALIGN Version 1.6                      Page 3

ALIGN PROGRAM COMMANDS

DRIVE  A..D

The DRIVE command specifies the letter of the drive under test:

A, B, C or D.  The default is drive B.  The DRIVE command does

not cause the drive to start.  Examples:

DRIVE A         (Set drive A)

D C             (Set drive C)

HEAD  0..1

The HEAD command selects the lower or upper head (diskette

side).  HEAD 0 is the lower head and the only head on a single-

sided drive.  HEAD 1 specifies the upper head on a double-sided

drive.  The HEAD command does not cause the drive to start.

Examples:

HEAD 0          (Set head 0, the lower head)

HEAD 1          (Set head 1, the upper head)

H 1             (Set head 1, the upper head)

TRACK  0..39

The TRACK command moves the head to a specified track and starts

drive motion.  The drive will continue to run until a key is

pressed or for 60 seconds.  (The 60 second run time may be

changed using the SECONDS command.) The track number that may be

specified is usually in the range 0 through 39, although the

upper track limit may be extended using the MAX command.

Examples:

TRACK 0      (Move to track 0 and run drive)

T 34         (Move to track 34 and run drive)

T 16         (Move to track 16 and run drive)

IN

OUT

The IN command moves the head to track 32 and then immediately

moves it "in" towards track 0 stopping on track 16.  IN is

equivalent to the TRACK 32 command followed by TRACK 16.

The OUT command is similar, but moves the head first to track 0

and then "out", away from track 0 stopping on track 16.  OUT is

equivalent to a TRACK 0 command followed by TRACK 16.

Both commands end up on track 16 with the drive running.  Track

16 is the radial head alignment track on the Dysan 40 track

alignment diskette.

ALIGN Version 1.6                      Page 4

After making a head adjustment it's a good idea to re-seek to

track 16 and check the cat's eye pattern again.  Both the IN and

OUT commands cause a re-seek to track 16.  The two commands

also allow checking the accuracy of seeks (and the resulting

head alignment) in both directions.  If there is a difference in

the cat's eye patterns in the two directions, you may want to

re-adjust the alignment to split the difference.  A difference

indicates play in the mechanism or an imprecise head stepper

motor.

GO

If the drive is stopped, the GO command simply starts the drive

at whatever track it happens to be on.  A null entry does the

same thing, so you can just press the RETURN key with nothing on

the command line to start the drive.

SECONDS  3..180

Normally, each time you start the drive using ALIGN, the drive

runs for 60 seconds if not stopped by pressing any key.

This is usually enough time to make a check or adjustment and

prevents the drive from running "forever".  The SECONDS command

allows you to change this 60 second limit anywhere from 3 to 180

seconds.  The SECONDS command does not cause the drive to start.

Examples:

SECONDS 120                (2 minutes)

S 10

S 30

CYCLE

The CYCLE command starts the drive and cycles the head through

tracks 0 through 3 in the following order 3,2,1,0,3,2,1,0,3,2...

and so on for the purpose of checking the track 0 switch.  The

drive changes tracks every few seconds so you have time to see

if the track 0 switch has switched.  In between each track

(0,1,2 or 3) the CYCLE command moves the head to track 16 so

there is a seek similar to that when a drive is first used after

power-on.

Believe it or not, on Tandon TM-100 series drives, the track 0

switch should switch between tracks 1 and 2.  (It's NOT supposed

to switch at track 0!) This is a position in the middle of the

first four tracks.  The actual track 0 signal on the drive

interface is a combination of the track "0" switch and other

indications in the drive electronics.

Switch here

|

V

0     1     2     3

ALIGN Version 1.6                      Page 5

While the CYCLE command is active, the PC's speaker sounds rapid

beeps equal to the current track number, 1 to 3, each time it

changes tracks.  Track 0 is a single longer beep.  These beeps

allow you to know what track the drive is on without trying to

watch a meter or test light and the PC's display at the same

time.  A higher pitch sounds in addition to the track beeps when

the switch should switch.  After a little practice, the sound

eliminates the need to watch the screen.  As you watch your

meter or light, don't be confused by the very brief switch

changes when the head moves out to track 16 and then back in.

While it's always nice to set things "right on," the track 0

switch adjustment isn't critical and the drive operates fine

even when it switches a whole track off from the ideal point

between tracks 1 and 2.

MAX 39..90

The MAX command sets the maximum track allowed with the TRACK

command.

Normally, the TRACK command accepts track numbers from 0 through

39.  The MAX command allows you to change the TRACK command's

high track limit all the way up to 90.  The MAX command does not

cause the drive to start.

There really isn't much occasion to use the MAX command.  It's

most obvious use is to allow the ALIGN program to be used with

80 track drives.  In this case you would specify MAX 79.  (Note

that the IN and OUT commands assume a 40 track alignment disk

and won't be useful with an 80 track alignment disk.  You can

still use the TRACK command to do the equivalent manually.)

Or, maybe you're looking for the track 39 signal on the DYSAN

alignment disk.  If it doesn't appear on what the system thinks

is track 39, maybe it will show up at what the system thinks is

track 40, or 41 or....

Actually, there is a limit to how far the head will go past

track 39.  You may want to use the MAX command to allow you to

investigate just how much further the head will go.  (A copy

protection scheme might use one track past the "end" of the

disk.)

However, before you go charging off into the stratosphere of

track addressing, realize that when the head hits the upper

limit, it does so with the full force of the stepper motor.

This alone could knock a drive out of alignment or possibly

cause permanent damage.  Use the MAX command at your own risk.

ALIGN Version 1.6                      Page 6

RESET

The RESET command causes a reset of the diskette system (via

BIOS INT 13 with AH=0).  RESET does not cause the drive to

start.

The next time any drive is accessed, it will then be

"recalibrated." This means that the diskette controller makes no

assumptions about which track the head is on, moves the head

until track 0 is located as indicated by the drive, and then

proceeds with the requested operation.  Once this recalibration

process is complete, the diskette controller keeps it's own head

position register and simply pulses the head forward or back,

one step at a time to get to a desired track.  If one pulse

is missed or the head is moved manually with power-on, then the

controller's track register won't agree with the physical

position of the head.

"Recalibration" is not an adjustment, but rather a normal

operation done by the diskette controller so it can get in sync

with the physical head position.  Every drive is "calibrated" by

the controller the first time you use it after power-on or boot.

The RESET command allows you to recalibrate during the drive

adjustment procedure.  For instance, assume the the drive is on

track 16 (or what you think is track 16).  If you issue a RESET

command and then simply start the drive motion (null entry), you

will hear the drive move to track 0 and then back out to track

16 as the drive is recalibrated.

The RESET command is useful when servicing drives and you think

the drive is not seeking to the correct track.  This could

happen while adjusting the track 0 switch.  Also, the drive

won't seek to the correct track anytime the head has been moved

against the stepper's electrical hold.  This could occur when

the head assembly hits it's travel limit at either end.  This

isn't normal operation, but when the adjustments are wrong it

can (and does) happen.

If you don't find an alignment disk signal where you think it

should be, try the RESET command.  It is also a good idea to use

the RESET command just before the final checks on radial head

alignment to be sure you're not exactly 1 or 2 tracks off.

ALIGN does an automatic RESET between every CYCLE command seek

to make track 0 switch adjustment easier.

Without the RESET command, it would otherwise be necessary to

re-boot or turn the machine off-and-on to force a recalibrate

operation.

ALIGN Version 1.6                      Page 7

INFO

The INFO command causes another screen to be displayed with a

brief description of each track on the Dysan 40 track alignment

disk.  It's simply for reference.  Pressing any key returns you

to the ALIGN command menu screen.

QUIT

The QUIT command terminates the ALIGN program and returns to

DOS.

DRIVE SPEED TESTING

ALIGN does not have the capability to check a drive's spindle

speed.  The IBM "Advanced Diagnostics" diskette that comes with

the IBM Hardware and Maintenance and Service book provides a good

software check of drive speed.  Lacking that, use the ALIGN

program to run the drive while you observe the strobe disk on the

bottom of the drive.

INSTANTANEOUS SPEED VARIATION TESTING

To test Instantaneous Speed Variation, the Tandon manual says to

first "write a 2F (all ones) pattern on any track." The ALIGN

program won't write this or anything else on your diskettes.  A

totally satisfactory substitute is to simply use the DOS FORMAT

command to format a work diskette in the drive under test.  This

puts a pattern on every track.  Admittedly, it's not "all ones"

but you'll hardly know the difference as you observe the pulse

jitter on the oscilloscope.  Also, while Tandon says to use "any"

track, the jitter is much worse on the inside (high) tracks.  Use

a middle track, such as 16, to check the specification.  Get to

know what normal jitter looks like on tracks 0 and 39 too.

ALIGN Version 1.6                      Page 8

TANDON TM-100 SERIES REPAIR TIPS

LOOSE SCREWS AND NUTS - HEAD MISALIGNMENT

In my experience, the most frequent cause of head misalignment and

drive failure on Tandon TM-100 series drives is a loose Allen head

screw on the stepper motor shaft.  This allows the motor shaft to

slip where it connects to the split band positioner pulley.

Tighten this Allen head screw before doing an alignment.  It's not

unusual to tighten it one-half turn holding the short end of an

Allen wrench with just your fingers.  Don't over-do it, however,

as you could strip the threads.

The stepper motor shaft is smooth and has no flats or keys.  As

long as you are doing an alignment and the Allen screw is tight,

the exact pulley position on the shaft usually doesn't matter.

But sometimes it can be out of the head alignment cam screw

adjuster range.  If this happens, loosen the Allen screw, rotate

the pulley a bit, tighten it, and try again.

While you're at it, check the two nuts that hold the track stepper

motor to the casting.  Also, remove the clear plastic cover on the

bottom of the head assembly and carefully tighten the little

Phillips screw that holds the split band on the pulley.

If you're not going to do an alignment, don't tighten these screws

since just tightening them might change the alignment some.

TRACK 00 SWITCH PROBLEMS - BOOT FAILURE

The track 0 microswitch frequently becomes intermittent or fails.

You can easily check this by powering-off the system and removing

plug 11 on the back of the drive.  Push a few pieces of bare wire

into the connector and check the switch contacts with an analog

ohm meter.  The switch is single pole, double throw (SPDT).

|     |  |

|     |  |  <- wires to switch

------------

|          |         COM - common center contact

| Plug     |         NC  - normally closed

|          |         NO  - normally open

|          |

Pin # ->  |4  3  2  1|

------------

COM     NC NO  <- Switch contacts

Operate the switch manually.  It is a little hard to get to and

you may want to remove the logic board first.  Check to see that

contact closure is positive, occurs when the switch clicks,

and without any wavering around.

If the switch is not operating properly, the problem is usually

just a very small amount of corrosion or oxidation on the switch

contacts.  Rather than replacing the switch, the easy and

ALIGN Version 1.6                      Page 9

effective repair is to pass some reasonable current through the

switch and operate the switch a few times.  Be sure that plug 11

is disconnected from the logic board and that you make connections

to just the plug.  Use an external CURRENT LIMITED low voltage

supply to burn off the little bit of oxidation.  I don't know how

much current to recommend.  I use just enough to cause a little

spark when the power supply leads are touched, probably 0.5 or

1.0 amp.  Be sure to use a current limited supply or put an

appropriate current limiting resistor in series.  If you don't

know how to do this, then find someone who does rather than risk

any harm or damage.

POSSIBLE SOURCES FOR SUPPLIES

For the type of drives usually used in the PC, the alignment and

service procedures are in the Tandon "OEM Operating and Service

Manual TM-100-1 and -2 Disk Drives, 48 Tracks per Inch."

Priority One Electronics sells this as part number 07TND179022001,

for $36.00.

Priority One Electronics

9161 Deering Ave.

Chatsworth, CA 91311

Telephone (800) 423-5922

(818) 709-5111

It is also Tandon part number 179022-001, listed at $25.00 on a

December 1982 Tandon price list.  I don't know if they will sell

it directly to end users.

Tandon Corporation

20320 Prairie Street

Chatsworth, CA 91311

Telephone (213) 993-6644

Analog Alignment Diskettes (AAD's) are available directly from

Dysan Corporation.

5.25" Double-sided, 48 TPI, 224/2A #800180         $50.00

5.25" Single-sided, 48 TPI, (numbers not known),   $25.00

(96 TPI alignment diskettes are also available.)

Dysan Corporation

5201 Patrick Henry Drive

Santa Clara, CA 95050

Telephone (800) 551-9000, Dept 50

(408) 988-3472

Extension or external diskette drive signal and power cables may

be difficult to locate or you may have to make your own.  Priority

One carries everything you need for the signal cable but not the

power cable. (If you know of a source for the male and female

power connectors in small quantities, please let me know.)