017/090 13 Jul 90 09:45:00

From:   Andy Vaught

To:     ALL

Subj:   LOADALL 3/4

Attr:

------------------------------------------------

Table 1.  LOADALL data area format

-----------------------------------------------------------------------

Originally included by Intel for chip testing, Microsoft is now using this

instruction in their RAM Drive program and in OS/2's compatibility box.

While this instruction is probably appropriate only for use in operating

systems and system-level utilities, it is important because it provides a

set of capabilities that are not otherwise available in a 286-based system.

We have received a copy of a document that describes LOADALL.  No company

name is shown, but is almost surely written by Intel.  The 15-page

document describes in detail how to use the instruction, so calling it

"undocumented" is not quite correct -- it is documented, but not in the

data sheet.  By restricting access to this documentation, Intel gives unfair

advantage to Microsoft and other large customers.

----------------------------------------------------------------------

Descriptor Cache Format

Bytes 0-2      24-bit segment base address

Byte 3         Access rights byte.  Format is the same as the access

rights byte in a descriptor, except that the "present"

bit becomes a "valid" bit.  If a "valid" bit is not

set, any memory reference using the descriptor will

cause exception 13 with error code of 0.

Bytes 4-5       16-bit segment size

GDTR and IDTR Format

Bytes 0-2      24-bit base address

Byte 3         0

Bytes 4-5      16-bit segment limit

----------------------------------------------------------------------

Table 2.  Descriptor Cache Formats

Intel's position is that this instruction is not useful to most users,

and can easily lead to machine crashes if not used properly.  Intel says

that they will provide documentation for the instruction on a "need-to-know"

basis, presumably only after a non-disclosure agreement has been signed.

Most developers, of course, would not know to ask.  The information we are

presenting here should be enough for you to evaluate the usefulness of the

instruction; if you intend to use it in a product, you should contact

Intel for the full documentation.

The opcode for LOADALL is 0F05 hex.  No operands follow the instruction;

LOADALL gets its data from a 102-byte block of memory starting at the

fixed address 800 hex, as shown in Table 1.  Table 2 shows the format of

the descriptor cache entries.  These are hidden registers that cannot

otherwise be modified by the programmer; they are set automatically when

a descriptor is read from the descriptor table.

ACCESSING EXTENDED MEMORY FROM REAL MODE

The following sequence is required to access high memory from real mode

using LOADALL:

1. Disable interrupts

2. Save the 102 (decimal) bytes starting at 800 hex.  (MS-DOS uses this

area for system code.  OS/2 presumably leaves it free for LOADALL,

but depends on application programs not to change it.)

3. Set up the 102-byte register image at 800.  The base address in the

data segment descriptor cache is set to select the desired high-memory

area.

4. Execute the LOADALL instruction.  The data segment now points to the

new, high-memory segment.

5. Move data to or from high memory.

6. Restore the base address in the data segment descriptor cache in the

image at 800, and execute another LOADALL.

7. Enable interrupts.

<<< The next step is not in the article, but conspicuously absent. >>>

8. Restore the original 102 bytes to 800.

Thus while it is appealing not to have to go to protected mode and back to

access high memory from real mode, this isn't much better.  LOADALL requires

195 clock cycles on a machine with no wait states, or 19.5 us at 10Mhz.

LOADALL can also be used to allow programs to be executed in high memory,

even though the processor is in real mode.  A special paragraph ID (such as

FFFF) is used to indicate when a program is running in extended memory.

Interrupt service routines must check the segment register to see if any

contain the special ID, and if so, they must reload the registers using

LOADALL to restore the segment base addresses before returning from the

interrupt.

--- Qtach2 v1.07 U8

* Origin: Blue Moon BBS >HST DS && T2500< Keep guns legal! (1:226/180)